Fred Goldstein   k1io

goldstein@carafe.enet.dec.com

Version 2.1  2-oct-1989

The Radio Shortest Path First (RSPF) Routing Protocol

For DDN Internet Protocol over Amateur Packet Radio

** DRAFT ARCHITECTURE -- FOR COMMENT **

** changes in V2.1 are noted by "**" and

should be edited out before final release**

CONTENTS

I.   Introduction and Version Notes

II.  Acquisition of router-router adjacencies

III. Acquisition of end node adjacencies

IV.  Link state propagation

V.   The Shortest Path First Spanning Tree

Appendix:  Router Parameters

I.  Introduction

Amateur packet radio use of the Internet Protocol does not yet provide

all of the capabilities of other IP networks.  One particular example

of this is IP packet routing.  Most existing versions of the AMPR IP code

make use of a static routing table.  This requires human intervention

every time a new backbone path is added, and adds geographic constraints

to address assignment which do not exist on the ARPA Internet. Some

implementations make use of automatic routing protocols (interior

gateway protocols, or IGPs) using distance vector routing.  These IGPs

were originally written for wireline networks and tend to scale poorly

to the amateur packet radio environment.

Many IP and other networks have implemented link state routing based upon

Dijkstra's "SPF" (shortest path first) spanning tree algorithm.  A

wireline implementation of SPF for IP is being standardized as the

Open SPF Interior Gateway Protocol (OSPF), and an SPF procedure is

being considered by ISO as the standard "IS-IS" routing protocol for

OSI connectionless networks.  A similar (and derivative) procedure can

be applied to AMPRnet (Net 44).  It is called Radio Shortest Packet

First (RSPF); this document outlines the RSPF protocol.

RSPF occupies the role traditionally referred to in TCP/IP networks as

an "Interior Gateway Protocol" (IGP), where "Gateway" means "router".

It makes use of the services of the Internet Protocol.  It is not

inconceivable that a router could use both RSPF and another IGP, or

communicate with another network using the Exterior Gateway Protocol

(EGP).  However these are not described in this document.

RSPF is intended to be implemented on routers, and need not be

implemented on end nodes for the end nodes to take advantage of

routing services.  Any IP station may be an end node giving no further

consideration to routing.

I.1. Elements of RSPF

The RSPF protocol is designed for use by internet-layer routing nodes (IP

Gateways) in a packet radio network using the DDN Internet Protocol.

It is comprised of four major functions:

1) Acquisition of router-router adjacencies

2) Acquisition of end node adjacencies

3) Link state propogation

4) Spanning tree route decision making.

Its net result is the automatic maintenance of a least-cost routing

table for use by IP Routing.  RSPF is optimized for the geographically

heirarchical addressing used in AMPRnet, but does not depend upon it.

RSPF is simpler than OSPF and IS-IS, as it is designed for PC

implementation within the Amateur Radio Service.  It also adds

procedures to take advantage of packet radio's "semi-broadcast"

nature.

I.2. Addressing convention

When an RSPF router communicates with an end node, it will typically

deal with a 32 bit IP address.  Routers themselves, however, also

support node group addressing (fewer than 32 bits need match).  This

follows the convention in the KA9Q IP routing program, which permits a

crude form of heirarchical addressing as well as allowing portable

operations to override the defaults.  RSPF looks for the match (node or

node group) with the greatest number of matching bits.  Only if the

number of bits matched is equal, then the lower cost path will be used.

Thus a match to a full node address will override a "cheaper" path that

matches its "class C net" of 24 bits, which overrides a "class B net",

etc., noting that AMPRnet does not follow strict 8-bit address

classification, and is a single Class A net.  In every case, a greater

number of bits matched is considered a superior path to a destination

than one that matches fewer bits, regardless of the value of the routing

metric ("cost").

I.3. Connection-oriented vs. connectionless lower layers

IP is a datagram network protocol, and supports both connection-

oriented and connectionless lower layers (subnets).  In a network with

a high packet loss rate, the local retransmission provided by a

connection-oriented datalink will substantially improve overall

throughput.  However, in a high-speed dedicated backbone, particularly

one implemented using full-duplex radio or wireline links,

connectionless links may provide better overall performance.  The

choice of which to use is a local matter; RSPF will work with both.

The reliability of the routing information, however, may be somewhat

greater with connection-oriented datalink procedures, since these will

give more rapid indication of a physical link failure.

I.4. Relationship to other protocols

The reliability of the network depends upon reasonably reliable

transmission of the routing update; hence, for non-broadcast procedures,

it is recommended that routers communicate with one another using

connected-mode AX.25, or another reliable data link layer.  (In any case

connected-AX.25 may be more useful than connectionless for backbone

links due to its local error correction ability.)

**CHECK THIS OUT FOR VALIDITY WITH ANDERS**

All packets specific to RSPF shall be exchanged inside IP packets using

a protocol identifier which, pending formal assignment of one, shall

be 73.  (How is this formally assigned?)  Such IP packets shall be

sent with a time to live (TTL) value of 1.  If broadcast procedures

are used, connectionless AX.25 UI frames shall be sent, with a

multicast address "QST-0" in the AX.25 address and an IP address of

44.255.255.255. (A router can also "read the mail" on passing radio

packets not addressed to it; such procedures are for further study.)

Note that in this document, "subnetwork" and "data link" are synonymous,

and refer to the layer over which IP packets are exchanged.

I.5.  Version 2.1 changes.

RSPF draft 2.0 was released in June, 1988, as the first nearly-

implementable version.  It was first implemented in September, 1989 by

Anders Klemets.  This version 2.1 reflects changes whose need was

discovered during this implementation.  These changes are both

editorial and, in a few cases, substantive.

The format of the Routing Update packet has been slightly modified.

In order to prevent fragments of two or more different routing update

messages from being erroneously merged, an Envelope ID is added to

each such packet, with the same Envelope ID on all fragments of a

multi-packet message.  The term for such a message is now "envelope";

it contains one or more "bulletins", each of which originated from a

single router.

There are no longer separate packet types for Full Routing Update and

Partial Routing Update.  Instead, they are distinguished by the value

of the subsequence number, which is always 0 for Full Routing Updates

and is never 0 for Partial Routing Updates.  A given envelope may

contain both types of bulletin.

Cost is now set on the basis of receiver instead of transmitter.  This

permits the automatic link quality adjustment to operate on the basis

of locally-received traffic.

The remaining horizon is stored in the links table.  This is needed

for consistency within the specification and was erroneously left out

of 2.0.

It is now explicitly stated that upon creation of a new router-router

adjacency, the routers exchage full routing information.  This allows

routers to initialize themselves with a reasonably complete map of the

network.

II.  Acquisition of router-router adjacencies

For RSPF to operate correctly, all routers must remain reasonably

current as to the state of the network at large.  This is handled by

the propagation of "bulletin" messages among routers.  End nodes need

not concern themselves with this; they will normally communicate

through one "designated" router at any given time, for all

(non-adjacent) destinations (not seen by ARP or other lower-layer

procedures).  End nodes can also, of course, connect to each other

directly, bypassing RSPF.

Each router maintains an adjacencies table.  Each router's adjacency

table lists the following information for all other nodes, both

routers and end nodes, from which it directly receives packets over a

subnetwork:

Adjacent node IP address (i.e., 44.56.0.44)

Adjacent node datalink (AX.25) address (i.e., K1IO-0)

Datalink used (i.e., AX0)

Datalink cost (i.e., 4)

Number of packets heard since last RRH update (i.e., 50)

Packet sequence number in last RRH update (i.e., 12593)

Time of last RRH update (i.e., 2130).

II.1.  Router-router hello

For the backbone to create its topology automatically, there must be a

way for routers to discover each other with minimal overhead.  For

this purpose, a router-router hello (RRH) message is provided.

Periodically (as an initial suggestion, shortly before beginning to

propogate the periodic links state bulletin to known adjacencies), the

router sends out the RRH message to the layer 2 multicast address and IP

multicast address.  RSPF makes no assumption of reciprocity (that

links are bidirectional), so receipt of an RRH packet provides the

receiver with information about a one-way (received) adjacency.

II.2.  Connection-oriented procedure

If a router uses connection-oriented subnet procedures to its own

adjacencies, then when a router receives this RRH packet, it checks to

see if it already has a link to that packet's originator in its own

links table.  If not, it waits a random period of time (initial

suggestion:  within the range of 0 to 10 times the link's value of T1,

DWAIT or SLOTTIME, and in any case much longer than the timers used

within a CSMA or Aloha subnet such as AX.25) and then attempts to

establish an AX.25 connection with the usual SABM; the router responds

with the usual UA (link established) or DM (link rejected).

If a two-way connection has been established, then both routers add the

link to their adjacency tables.  While the existence of this route is

set reciprocally, the cost of each side of the route is set separately

for each side of the connection.  Cost is propagated in the routing

update (link state) packet.  Thus the adjacency between two routers is

not actually used for real traffic until the first routing update

packet exchange has taken place.

Loss of an adjacency may then be noted by the loss of the subnet

connection.  When this occurs, the router removes the adjacency from

its adjacency table and follows the "bad news" procedures outlined

below for link state propagation.

II.3.  Connectionless procedure

If a router uses connectionless datalink procedures to its own

adjacencies (suitable to low-loss links), then when a router receives an

RRH packet, it checks to see if this adjacency is already in its

adjacency table.  If not, then it is added.  It also sends RRH packets

with the same frequency as with connection-oriented subnets.

Loss of an adjacency may be noted by timeout; if no RRH message is

received, and no frames have been received from the adjacent router for

a period of time (initial suggestion:  slightly over twice the maximum

interval between RRH messages), then the adjacency becomes suspect.

The router should attempt (**a settable number of times**) to exchange a

packet (ICMP echo) with the suspect adjacency; if unsuccessful (after

the usual number of retries), the route is marked lost.  It may also

be marked lost if other attempts to send data through that router

fail, such that the implementation determines that there is a high

probability that the link is lost.

Table II-1.  Coding of the RRH PDU.

1	 	  2

|0              |8              |6              |4              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| RSPF Version #| Type (RRH)    | Checksum                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         Full IP Address of sending router                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  last packet sent seq. #      |  flags        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|        plaintext                                              |...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Parameters--

An RSPF Router-Router Hello packet is sent within IP with a type

of <tbd-- 73 suggested>.  Each RRH packet contains the following

fields:

RSPF Version Number:  Version number of this protocol (initially 1).

Type:  Value of 3 for RRH.

Checksum:  IP-style checksum

Address:  Full IP address of sending router

Last packet sent sequence number:  An integer (mod 65535)

incremented by 1 for every frame sent by the datalink layer across

this interface.  This value allows receiving entities using

connectionless procedures to use the automatic link quality

measurement technique described in II.4.

Flags:  The low-order bit is 1 if connectionless datalink is

preferred; 0 if connection-oriented is preferred.  (Set by

system management based upon anticipated link quality.)

Other bits are reserved (sent 0).

Plaintext:  An optional text message (length may be up to maximum

size remaining in datalink PDU).  This might serve, for example,

to "broadcast" the router's existence to persons who might be

"reading the mail" (monitoring a radio channel promiscuously).

II.4.  Automatic link quality measurement

A connectionless link or subnetwork may have very reliable, or very

sporadic, performance.  Since there is no procedure for ensuring the

reliability of packets sent over a connectionless link, a high rate of

packet loss may occur without being detected by the routers.  If this

loss is high enough, another route may become a better choice; a high

enough packet loss rate may be enough to mark a link as "down".  The

automatic link quality measurement procedure allows links which are

not yet "down", but whose performance is substandard, to be noted.

Every router shall maintain, for each link, a count of all packets

sent over each link.  Every time an RRH message is sent, it includes

the current value of this counter (modulo 65536).  Every router also

maintains, in its adjacency table, a count of the total number of

packets received from said adjacency since the last RRH message, and

the value of that counter as received in the last RRH message.

Upon receipt of an RRH message, the recipient router compares the value

of the received packet counter with the last received value in the

adjacency table.  The difference (taking into account wrap-around at the

modulus) is compared with the number of packets received since the last

RRH message.  (This works even if an RRH message is lost.)  This packet

loss ratio is then maintained as a guideline to determine link quality.

If link quality falls below a settable threshhold, the link is

suspect.  Timestamp can also be used to compute packet arrival rate.

Connection-oriented data links presumably deliver 100% of attempted

packets.  A high-quality connectionless link, such as Ethernet/LLC1, will

come close to this.  However, single-frequency packet radio links are

prone to packet loss for several reasons, including hidden transmitters,

lack of collision detection, and rf interference.  The packet loss ratio

is useful in setting link cost, and may also be used to determine

whether a link should use connectionless or connection-oriented

procedures.

If a router reports, in its link update packets, that a given link has a

cost of _n_, then its adjacencies (and only its adjacencies) may apply

the packet loss ratio to adjust the cost which they maintain in their

link state tables.  These adjusted costs, rather than the received

costs, may then be propagated to other routers.

Such procedures should be applied sparingly, as each change must be

propagated and could, if used too frequently, result in network-wide

instability.  A suggested (experimental) algorithm is as follows:  A

percentage threshhold x is set, as is a cost-adjustment factor y.  If

fewer than x% of packets are received during a measurement interval,

the cost of that adjacency is multipled by y.  For example, if x is 80 and

y is 1.5, then an adjacency with a nominal assigned cost of 4 will be

up-costed to 6 if only 70% of packets are received, but will be

restored to 4 if 80% or more are received during the next measurement

interval.  The measurement interval is the time between RRH messages,

which precede routing update messages.

III. Acquisition of end node adjacencies

Three possible means of determining adjacencies to end nodes are the use

of connected-mode AX.25 links, the use of ARP, and the use of a

"wiretap" algorithm (see RFC981).  Unless a connection mode Data Link

layer (with keepalive timers) is used, adjacent nodes may need to send

each other messages at regular intervals (ping) to ensure that the

link is still usable.  A procedure is outlined below for routers and

end nodes to acquire knowledge of each other.

It is assumed that RSPF will not be activated in end nodes; this

permits them to use simple version of the IP software.  A node that

has RSPF support in its software but operates as an end node can also

use the router-router connection procedures and simply broadcast its

adjacency to the router in a one-entry bulletin with a horizon of one.

Such a node may also be simultaneously homed on two or more other

routers, unlike true end nodes whose traffic either bypasses RSPF

(using ARP) or arrives by way of its associated router.

There is no "redirect" function provided in RSPF.  Since radio does

not provide a true "broadcast" topology subnetwork, a router cannot

presume that if both end nodes can hear it, that both end nodes can

hear each other.

If an end node knows the IP address of the router which will connect

it to the network at large, it may establish a connected-mode AX.25

(or other subnet) connection to the router; the presence of this

connection indicates that the node is reachable from that router,

which then adds it to its links table and subsequent bulletins.  This

may, of course, require an ARP exchange in order to acquire the AX.25

(or other data link layer or subnet) address.

Alternately, the end node can simply use ARP and use connectionless

link procedures. In this case the router should assume that the end node

is available until either a rather lengthy timer expires, or the router

is unable to make an ARP contact after the ARP timer expires.  (A loss

of reachability should not be inferred from the ARP timeout.)

Routers should periodically broadcast their availability (suggested

interval:  every 15 minutes) with an AX.25 UI frame sent to QST-0 (the

AX.25 broadcast address).  A human-readable "unproto" message may go

here, allowing individual operators to recognize routers and connect

as appropriate.  (No specific PDU coding is provided, as the end

nodes do not use RSPF, and thus this is not really an RSPF packet.)

A router may also choose to use "Promiscuous ARP" to provide service to

an end node which is attempting to connect with an IP address reachable

by the router.  In such a case the router should wait an extra interval

after receiving the ARP request because the desired destination may

actually be directly reachable; ARP procedures may need to be modified

to provide this.

Another potential approach is for routers to simply listen to AX.25

traffic on the link and determine who is adjacent to whom.  This is the

gist of the "wiretap" algorithm in RFC981, which also finds non-adjacent

nodes by taking advantage of the source routing found in AX.25 frames.

Integration of wiretap into RSPF is for further study.

IV.  Link state propagation

Link state information is propagated between routers within bulletin

envelopes, which are sequences of packets containing partial or full

copies of the sending node's link state table.  Both point-to-point

and broadcast procedures are provided.

IV.1.  Optional multicast/broadcast

Packet radio is inherently a broadcast medium.  Packet radio networks,

however, may be viewed as a collection of individual links which happen

to use a broadcast physical medium.  It is also possible to exploit the

broadcast nature of the medium.  RSPF link state propagation procedures

allow but do not require such multicasting.  If the link uses

connectionless procedures for user data packet exchange, then broadcast

procedures should be used for link state packet exchange.  The converse

may not necessarily be true:  The threshhold of loss that militates

against connectionless transmission of user data may be more stringent

than that which call for non-broadcast transmission of link state

packets.  (Optimal parameters are for further study.)

IV.2.  Routing update bulletins

Routing updates are passed along from router to router via routing

update bulletins, which are broadcast within routing update envelopes.

Bulletin propagation is designed to make it extremely likely that

every node within a given "horizon" receives every routing update

message sent out by a given node.

Every router originates information about changes in its own adjacencies,

as well as periodic retranmissions of its entire list of adjacencies.

These bulletins are then propagated among other routers. The router whose

adjacency information is being broadcast is called the _reporting

router_; this may be some hops away from the routers which forward it to

their own adjacencies.  Each reporting router's bulletins (adjacency

updates) contain a sequence number (and in some cases, a subsequence

number).  These sequence and subsequence numbers are generated by the

reporting router and propagated; they are not changed when a bulletin

is relayed.  They are incrememted by 1 (modulo 65536) every time a new

one is generated.

Bulletins may also carry change information incremental to previous

bulletins.  These carry the same sequence number as the full routing

update bulletin to which they are reporting incremental changes; each

such partial routing update bulletin has a subsequence number.  The

subsequence number is zero for a full routing update bulletin.

Every bulletin also has a horizon value, set by the reporting router,

associated with each of its constituent links.  (A given reporting

router may have more than one constituent link, if it is a multi-port

router.)  Every time a bulletin is propagated, each horizon value is

decremented by 1.  When it hits zero, the bulletin is not propagated

further.  Note that for horizon purposes, a bulletin's individual

constituent links may have separate horizons; when a link's horizon hits

zero, other links' adjacency information from the same reporting router

may continue to be propogated.

It should also be noted that if a given link has adjacencies with

different horizons, these must be treated as separate links, since

horizon is reported as a characteristic of link.  Such a circumstance

may occur, for example, when a router serves a node group.

Adjacencies within the node group are typically propagated with short

horizons, since they are only of local interest (i.e., to other nodes

in or near that node group).  Similarly, the node group itself is

propagated with a long horizon, across a backbone.  However,

adjacencies outside the node group may be propagated with long

horizons, as they would not otherwise be reached across a backbone

dependent upon node groups for long-haul routing.

Every router maintains within memory a routers table containing one

tuple for every other router on the network.  This tuple contains the

following elements:

IP Address

Last received bulletin sequence number

Last received bulletin subsequence number

Timestamp for when the data was received.

Horizon remaining in bulletin when received **NEW IN 2.1**

This table is used to coordinate the receipt and transmission of

bulletins, using either broadcast or non-broadcast procedures.

IV.3.  Flooding without congestion

Upon initialization of adjacencies

Bulletins are forwarded in a routing update envelope which may contain

one or more reporting routers' bulletins (adjacency lists), which are

flooded to the network.  A bulletin envelope may actually concatenate

multiple reporting routers' bulletins; this is in fact the typical

case, when routing update packets are exchange on schedule, **and when a

given router acquires a new adjacency**.  However, partial routing

updates (good news and bad news) may be sent at any time.

The first time an adjacency is acquired, the two routers perform a

full routing update with each other, exchanging their complete link

lists.  Once this is complete, the routers perform the SPF algorithm

and compute new routing tables.

Preventing loops and retransmissions

A bulletin from reporting router X (which lists all of X's

adjacencies) is sent, initially by X, to every adjacent (to X) router

A.  This router A passes it along to all of its own adjacencies B,

etc..  This flooding is controlled such that no reporting router's

adjacency update is sent more than once between the same pair of

routers.

After router A sends its bulletin envelope to B, the recipient router

B then looks at the sequence number of each reporting router X's

adjacency bulletin within that envelope, and for each reporting

router's bulletin, compares the sequence number of the just-received

adjacency bulletin with the highest sequence number previously

originated from X.  If the just-received bulletin is newer (higher

number), then it sends a positive acknowledgement to A, and joins in

the flooding to its own adjacencies.  The horizon is, of course,

decremented.  ** HOW IS IT POSITIVELY ACKED?  NEED WE? **

If the bulletin from X has the same number as was stored in B, B checks

the horizon left in the received bulletin against the horizon left

when it previously received the bulletin.  In the event that the

latest bulletin received had a shorter (lower number) horizon left

than the earlier one, it simply acknowledges the (redundant) message.

But if the reporting router X's horizon left is greater for the new

copy of the bulletin, router B propagates it as if it were a new

bulletin. This way, if the bulletin happened to first arrive via a

roundabout path, it won't accidentally fail to reach the intended end

of its range. (Summary:  Newer or longer-horizon bulletins are both

passed along.  Same age or older (by sequence number) or same or lower

horizon will not be propagated.)

If any router B receives from router A a bulletin (from any reporting

router X) that contains a lower sequence number than one that had

previously arrived at B from said X, then it can be presumed that A

has "obsolete" information.  B replies by sending a bulletin to A with

the latest link state information for that node X.  As a corrolary, a

router may poll for information about a given reporting router by

sending a routing update bulletin for that reporting router with a

sequence number that is lower than the latest one it actually has

received.  Said bulletin may contain zero links' information.  (Note

that since the sequence number is modular, a value of 0 is not correct

for this function as 0 is higher than 65535; the "poll" number should

be only slightly lower.)

IV.4.  Non-broadcast bulletin envelope exchange

A router may acquire routing information from adjacent routers via

point-to-point procedures which treat every adjacency as a separate

logical data link.  (Such a procedure also works, of course, over

point-to-point links such as wirelines.)  This tends to have the highest

reliability, since connection-oriented data link control procedures can

be used to ensure the accuracy and completeness of the data passed over

the link.  It has the disadvantage of requiring separate transmission of

the same data to different adjacent nodes on the same radio channel.

Bulletin envelopes are thus exchanged (when connection-oriented is

selected) periodically (based upon timers) and when new information is

received (over a new adjacency, and when good and bad news arrive).

Delivery of these bulletins is considered to be gnerally reliable.

IV.5.  Broadcast bulletin propagation

When a router is adjacent to several other routers via the same

broadcast (i.e., packet radio) channel, retransmission of routing

bulletins to each adjacency makes additional use of bandwidth, which may

be a scarce resource.  A broadcast procedure may be used to pass the

bulletin along that link.  Note that broadcast propagation of bulletins

may be combined with non-broadcast propagation, on a link by link basis.

Although packet radio is a broadcast medium, it is not a perfect one:

The reliability of multicast packets is not as high as for wireline LANs.

This leads to the need for a unique broadcast "flooding" technique.

A routing update bulletin is broadcast to the Layer 2 multicast AX.25

address using the IP multicast address.  Individual recipient routers

may or may not receive the entire bulletin, since there is no

acknowledgement provided.

In the previously described case of a non-broadcast message sent via a

connection-oriented datalink, the entire routing update bulletin can

be assumed to have been received intact.  Thus, if a given reporting

router has originated a new complete list of its adjacencies (new

sequence numbers, subsequence numbers are 0), then any adjacency not

included is presumed to have ceased to exist.  Such a presumption is

not always possible when a bulletin was received via unacknowledged

broadcast, as the message might have been received in part.  This

should not make the partially received bulletin unusable.

A bulletin envelope is transmitted in one or more packets, each of

which constitutes a numbered fragment of the whole bulletin envelope.

Within the transmitted bulletin envelope, each individual reporting

router's bulletin begins with a node-header which contains the number

of links being reported on.  Within each bulletin, each link is

identified by a link header, each of which specifies the number of

adjacencies being reported on (for that link).  Since each IP packet

that makes up a bulletin contains a fragment number, it is also

possible to determine whether or not a fragment was lost.  Each packet

also contains an envelope-ID, which prevents accidental mis-assembly

of different bulletins that may arrive close in time. **NEW IN 2.1**

In connection-oriented non-broadcast procedures, a routing update

bulletin (not a partial one with a subsequence numbers >0) provides a

complete list of adjacencies known to the sending node, except where the

horizon is exceeded.  Absence of a previouly-known adjacency then

implies that the adjacency has been lost.  However, that assumption does

not apply to fragmented bulletins received in part, which can occur with

broadcast procedures: If a fragment was lost before reaching the end of

a given reporting router's bulletin within the bulletin envelope, then

the absence of a previously-known adjacency in the received bulletin

does not mean that the adjacency was lost.

RSPF procedures dictate that routing update bulletins with a subsequence

number of zero are presumed to be complete lists of adjacencies from

their originators; higher subsequence numbers represent incremental

changes only.  In the broadcast procedures, a routing update bulletin

with a subsequence number of zero, if received only in part, is

treated as a partial routing update bulletin.  If it received in full,

it is treated as a full routing update bulletin.

Thus, the recipient compares the sequence number with the previously

received sequence number from that reporting router.  If it is higher

than the previously received one, then its adjacencies are used.  If

it was received in full, and had a subsequence number of 0, then its

previous adjacencies are replaced (removed if not contained in the

latest full routing update bulletin).  If it was not received in full,

or if its subsequence number was not zero, then its previous

adjacencies are left intact unless explicitly changed by the received

bulletin.

If a bulletin is received in full, then the routers table is updated

with the latest sequence and/or subsequence number, horizon **NEW 2.1**

and timestamp.  If it is received in part, then these entries are not

updated.  After the bulletin has been completely transmitted, a

recipient node which has received a partial update from a reporting

node may poll for that node's latest information, by using the (now

known to be obsolete) sequence number for that router in its router

table in a bulletin, with zero links for that reporting router.  (This

is the same polling procedure used for non-broadcast links.)

Note that if a fragment is lost, a reporting router whose node-header is

in the lost fragment will of course remain unchanged in the recipient's

data base.  Furthermore, any data received in subsequent fragments,

prior to a node-header, will also be meaningless.  The last adjacency

of the last link in a reporting router's bulletin will have the Last

flag set to 1, signaling that following the address, a node header

follows.  Note also that the first node-header within an envelope is

pointed to by the sync byte in the envelope header.  The last flag and

sync byte should match or an error should be noted.

IV.6.  Routing update bulletin packets

A routing update bulletin envelope (Table IV.1) may contain several

different reporting routers' updated link state information,

concatenated into one message, with a different sequence number for each

source (reporting router).  One of the sources, of course, may be the

local router.  Each router's link state information is further broken

down by link, which allows its backbone routing information to be

propogated separately from its local end node adjacency information.

Incremental changes (good news and bad news)

Bulletins that only report changes in state come in two flavors.  Good

News bulletins inform other routers that an adjacency has been noted.

Bad News bulletins inform them that an adjacency has been lost.  If an

end node establishes a connection with a router whose node group default

addresses (based on the significant bit count) already include that end

station, however, no bulletin need be sent out, as packets to that end

station will already be routed correctly.  Theoretically, a router could

send out a new full routing update bulletin every time it gained or lost

an adjacency. However, the use of shorter Good News and Bad News

packets, which do not contain a full routing update, allow the network

to remain relatively current with less transmitted traffic.

Good news and bad news packets are propogated like other packets,

but are not originated by the same rules.  While full routing bulletins

are originated based upon a timer, good news packets are transmitted

immediately upon receipt and initiated immediately after an adjacency

is initialized.  This enables new links to be useful quickly.  Bad news,

however, should not travel as fast:  A node should cache any bad

news message for a time (initial recommendation for this timer: 60

seconds) while waiting for the link to come back up.  This helps keep

the network stable; if the node receives a packet destined for the

lost destination, it may send an ICMP "host unreachable" message

to the originator of the packet, unless it can reroute the packet

itself (as may happen with the loss of a backbone link when others

still exist).

Because good news and bad news messages represent changes to the last

full link state bulletin propogated, but do not purport to completely

represent a node's link states, they carry bulletin subsequence

numbers.  These use the last bulletin sequence number originated by the

reporting router, but the sub-sequence value increments from 1. (A full

link state packet has a sub-sequence value of 0, and resets the

subsequence counter.)

Routes to nearby destinations

Sometimes more than one router will serve the same area (determined by

the significant bits' matching), and they will need to know which one has

the better path to a given station.  These adjacency messages may only

require a short horizon, as will Good News and Bad News packets which

refer to end nodes going on and off the air.   Higher horizons are

needed for backbone routers.

Table IV.1.  Routing update (link state packet) bulletin envelope:

1	 	  2

|0              |8              |6              |4              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ----

| RSPF Version #| Type          | fragment #    | fragment total| packet

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ -hdr

|       Checksum                | sync byte     | # nodes below |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Envelope-ID                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ----

|         Reporting node #1 full IP Router-Address              | node

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ -hdr

|  Node 1 bulletin  sequence #  | sub-sequence #| # links       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ----

| horizon left   |  ERP factor  |  link cost    |  #adjacencies | link

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ _-hdr_

|significant bits|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|              Adjacent node(s) 1,1,1 IP address                | adj.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ---

|significant bits|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             Adjacent node(s) 1,1,2 IP address                 | adj.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ---

...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|significant bits|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             Adjacent node(s) 1,1,n IP address                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ---

| horizon left   |  ERP factor  |  link cost    |  #adjacencies | link

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ---

|significant bits|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             Adjacent node(s) 1,2,1 IP address                 | adj.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ---

...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|significant bits|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             Adjacent node(s) 1,2,n IP address                 | adj.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ---

|         Reporting node #2 full IP Address                     | node

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Node 2 bulletin sequence #   | sub-sequence #|  # links      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ---

| horizon left  |  ERP factor   |  link cost    |  #adjacencies | link

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ---

|significant bits|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ adj.

|             Adjacent node(s) 2,1,1 IP address                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ---

|significant bits|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             Adjacent node(s) 2,1,2 IP address                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| horizon left    |  ERP factor |  link cost    |  #adjacencies |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|significant bits|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             Adjacent node(s) 2,2,1 IP address                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|significant bits|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             Adjacent node(s) 2,2,n IP address                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         Reporting node #n full IP address                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Node n bulletin sequence #   | sub-sequence #|   # links     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

etc.

Parameters--

An RSPF bulletin packet is sent within IP with a type of <tbd - use 73

until an official value is assigned>.  Each routing update envelope

contains an envelope packet header that contains:

RSPF Version Number:  Version number of the protocol (initially 1).

Type:  (Value 1 for Routing Update Bulletin Envelope)

Fragment Number:  States which fragment, in a segmented message,

this is, beginning at 1.  Non-fragmented messages use 1.

Fragment total:  Total number of fragments in message; 1 if not

fragmented.

Checksum:  IP-style checksum.

Sync byte:  Which octet in this packet (counting from this

byte as byte 0) is the beginning of the first node-header.  If 0,

this fragment has no node-header.  Non-fragmented messages

use a value of 4 (because 3 bytes follow in packet header).

**NOTE CHANGE IN 2.1**

Number of nodes reporting:  The number of reporting routers in the

messages that follows (this packet or a sequence of packets forming

the envelope).

The node-header (for each reporting router) contains 8 octets:

Reporting router #n full IP router address:  The IP address of

the router whose adjacencies are being reported below.  (Note

that if a router uses separate IP addresses on its links, it

should still adopt a single one as its router address.)

Bulletin sequence number:  When a bulletin is passed along, this

number is NOT changed; every new bulletin from a given Reporting

router has a value 1 higher than the previous bulletin from that

reporting router.  (Note:  This is modulo 65536, so implementations

must cope with the "wrap around" and consider values below, say,

100, to be "higher" than values above, say, 65400.  Between 100

and 65400, modular arithmetic is NOT used.)

Sub-sequence number:  Good news and bad news packets representing

incremental changes from the last full report increment this value

by 1; it is 0 for full bulletins.

# links:  The number of different cost-horizon values (typically,

but not necessarily, representing different types of link in a

mulitiport gateway) being reported below; the following four octets

are the header for each link.

[For each reporting router, adjacencies are reported separately by

link cost.  This is the received cost value for that data link, after

any adjustment that may be based upon packet loss ratio.  Adjacencies

are also reported separately by horizon, even if the cost is the same.

It does not matter at this point if there are multiple RF or other

links if their cost and horizon are the same.  Likewise, separate

received costs or horizons on one link will be treated separately

and such adjacencies will be grouped under separate link headers:]

Horizon left:  This number is decremented every time a routing update

bulletin is passed along; when it reaches 0, it is not passed further.

Link cost:  A "figure of merit" for each link, rising with

slower/poorer links.  This is the number whose total is minimized

by SPF.  The range is 1-127.  As a starting point, a 56000 bps fdx

backbone link might have a value of 2, a 4800bps hdx link a value

of 5, a 1200bps hdx link a value of 10 and a 300 bps hf "wormhole"

a value of 20.  An Ethernet or high-speed (1 Mbps+) link might

then have a value of 1.  A value of 255 denotes the loss of a

link; this is found in Bad News packets.  These values should be

coordinated network-wide;  adjusting them will change the way

packets are routed by RSPF.

Number of adjacencies:  The number of adjacencies to be listed for that

reporting node.

ERP Factor:  Used for "approximating" a route; contains the number of

significant bits for which a given node can be presumed to be a valid

router, even if it doesn't report in detail.  A low factor = wider

coverage area; thus ERP of 16 means that if NO other match is found for

a given address, this router will try to handle it if it matches 16

bits.  Basically a handle for future enhancements; its use will not

be required in the initial release of RSPF.

For each adjacency of the given link cost, the following is provided:

Significant bits: The number of bits used for node group routing

purposes.  Usually 32 but may be lower if a given link purports to

serve all end nodes in an area defined using the most-matched-bits

node group convention.  Higher numbers of bits matched take a higher

priority than least cost.  This uses the low-order 5 bits of the

octet.

Last-flag:  If this is the last adjacency in the list for this

reporting router, this value is 1; otherwise it is 0.  (This

occupies the high-order bit of the significant bits octet.)

Full IP address: The full IP address for this node.

Note that the n,n,n vector within the bulletin has three fields in the

above representation: Reporting router within bulletin envelope, link

cost/horizon within reporting router's bulletin, and reporting adjacency

with that link cost/horizon.

IV.7.  Fragmentation

In a moderate to large network, a full routing update can easily exceed

the maximum size of an AX.25 frame or IP packet.  The RSPF Routing

Update message, however, may be sent in fragments.  The IP

fragmentation function is not used for this; bulletins are not assumed

to be terminated by a packet boundary.  Each fragment is, however,

numbered in the packet header, along with an indication of the number

of the total number of fragments in that envelope.

In order to permit parsing of the incoming fragments by routers who

are using unacknowledged broadcast information (with the high

likelihood of lost fragments), every bulletin's packet header contains a

sync byte indicator.  This indicates which byte, where the next byte in

the header is byte 1, is the beginning of a node header.  If a previous

fragment was lost, the receiver should ignore the number of bytes

indicated in the sync byte before resuming parsing of the packet.  (If

the fragment does not exceed 255 bytes, this will always be sufficient.

It is recognized that long packets may not be able to use this mechanism

reliably, and a value of "0" should be used to indicate that no sync is

possible within this fragment.)

Each routing update bulletin envelope takes the form of a three-

dimensional list, with the dimensions being reporting router, link and

adjacency. A given bulletin envelope may report on link state from one

or more remote nodes, as well as from the sending node.  Each node may

have one or more links; each link may have one or more adjacencies.

Packets may not be fragmented within adjacencies, but may be

fragmented after an adjacency's address and before the next adjacency's

significant bits field.  (This presents a 5-octet field that should

not be fragmented.)  The next fragment, in a new packet, simply begins

where the last one left off; the receiver knows how much more to

expect based upon the node and link count in the respective

node-header and link-header.

A router may not start sending a new Routing Update message of any kind

to any given IP address until all fragments of a previous message to

that address have been transmitted.  This applies both to point to

point (non-multicast address) and multicast procedures.

IV.8.  Bulletin Timers

The timers used for bulletin updates must be a compromise between

maintaing the network's current state and the traffic required to do

so.  An initial suggestion:  Good news messages should be initiated

within a few seconds and bad news messages should be initiated within

one minute, with relatively short horizons on the bulletins (i.e., the

routers within the region).  Full routing updates with normal horizons

should be sent out every 30 minutes.  If the network is small, more

frequent updates may be possible; too frequent updates risk choking

the network with update traffic.

V.  The Shortest Path First spanning tree algorithm

As a routing node comes onto the network, it acquires over time a

complete list of adjacencies between other nodes on the network except

as limited by the "horizon".  Each adjacency (point to point link)

within the entire known network has a "cost" associated with it.  It

should be noted that adjacencies, for the purposes of this algorithm,

are "logical" and not necessarily physical;  if a subnetwork link

occurs below IP (as in AX.25 digipieating or NET/ROM), the two ends of

the link are still adjacent.  (Adjacency at the IP internet layer is

based upon subnetworks, which may contain their own links.)

Cost is set for the receive (** Changed in 2.1 to facilitate automatic

link quality adjustment **) side of each link.  If the receiver and

transmitter do not agree on cost, the network shall apply different

routes for packets going in opposite directions between the same two

end nodes.  (This is not a problem.  In a radio environment, one

should NOT assume reciprocity across a link.)

V.1.  Tables

Each router builds a _link state table_ that lists, for every known

link in the network (from every reporting router), the two ends and

the cost of that end of the link.  The ends are listed by an IP router

address and, for the destination IP router address, a number of

significant bits.  This is what's updated by the bulletins and by good

news/bad news messages.

Source IP address	Dest. IP addr/bits	Cost

44.56.0.44		44.56.0.128/32		5

44.56.0.44		44.56.0.12/25		10

The goal of the SPF algorithm is to build a _paths table_ which lists,

for every reachable node (or node group approximation of fewer than 32

bits) on the network, that node address (or node group address and

number of significant bits), the adjacent node used to get there

(i.e., where you blast the packets to next), and the total cost of the

path.  (This feeds the Route table in the IP Route module in NET.)

This is done by building a spanning tree with the router doing the

computation (the home router) as the root of the tree.  The paths

table thus lists the best way across the tree from the home router to

all known destinations.

Every router contains, for the purposes of building the tree, a

_trial table_ that has three entries:  Destination address/bits,

adjacent node, and cost of this path.  The paths table additionally

has one more entry, the _parent_ node, which is the last hop

before the destination.  Thus in a path A -> B -> C -> D -> E, home

router A views E as the destination, D as the parent, and B as the

adjacency.  Note that in the path from A to B, A is the parent node.

Begin building the paths table by using the home router as the first

node on the paths table.  The cost to reach yourself is always 0, so

make the first entry on the paths table as follows:  Source=self,

destination=self, parent=self, and cost=0.  From here on in, parent

is always (by definition of the SPF spanning tree) the node most

recently added to the paths table.

Destination	Adjacent 	Parent		Cost

44.56.0.128	44.56.0.128	44.56.0.44	5

44.56.0.131	44.56.0.128	44.56.0.128	10

44.56.0.200	44.56.0.128	44.56.0.131	15

Paths Table showing relationship between

destination, parent and adjacent nodes, where the home

node is 44.56.0.44 and 44.56.0.200 is three hops away;

all hops here have a cost of 5.

V.2.  Scanning the links

The home router now scans its links table looking for all nodes (routers

and end nodes) that are adjacent to the current parent node, except

for links to nodes which are already on the paths table.  (It is

generally fastest to build the paths table by scanning the links table

in order of increasing link cost.)  Each of these new nodes is added

to the trial table, except for the parent node (which is already on

the paths table, so it can't possibly have a shorter path).  The value

of cost placed on the trial table is the cost of the link from the

parent to the destination plus the cost from home to the parent node

(which value is already on the paths table).

A node may only appear once in the trial table at any given time.  If

an adjacency is found to a node that is already on the trial table, a

new entry replaces the existing entry if and only if the new total

cost is lower.  If the cost is higher, it is ignored.  (If the costs

are equal, then a "tiebreaker" is determined by treating the

lower-numbered IP address as the lower cost.  This will simply make

the spanning tree more deterministic in case of tie.)  Thus the trial

table always contains the lowest cost path to each destination found so

far.

Once all of the links from the parent node have had their chance to go

onto the trial table, scan the entire trial table for the _one_ entry

with the lowest total cost.  This not need be a link from that parent

node!  In case of tie, pick the lower IP address (again just to be

deterministic).  Move this node to the paths table;  guaranteed,

there'll be no cheaper path to that node!  The adjacency used for that

node in the paths table is the adjacency to its parent node.  Note

that the parent _must_ already be on the paths table since that's the

source of the parent; you're working your way outward.

This new addition to the paths table becomes the new parent node.

Repeat the procedure above (from the beginning of V.2) until there are

no nodes left on the trial table.  This means you've completed the

spanning tree and have found the least-cost path to every other node.

One of the router parameters is maximum_cost.  If the cost to a given

parent node exceeds this value, the procedure stops, since all

subsequent entries in the route table will have a higher cost.  This

value has some semblane to the time-to-live parameter of the IP

packet:  It makes little sense to compute a routing table to nodes

that cannot be reached within time-to-live.  (Ideally, ttl will be

implemented using a timer rather than a node counter, but this is

difficult to do with some of the packet radio data link controller

implementations; vis., TNCs.)

A router should recalculate its routes periodically based upon the

current links table.  How frequently depends upon the CPU load involved.

Initial estimates are that it should be recalculated after receipt of

every routing update bulletin:  Partial calculations do not save

enough CPU time to make them worthwhile.

V.3.  Filling in the routing table

The route table in NOS and NET (the KA9Q et al implementations of IP)

contains, for each entry, the destination address, number of bits,

interface, gateway and metric.  This is essentially left intact,

except that metric is filled in by cost and the routing decision looks

for the least cost of all matches.  The route table is filled in from

the paths table.

Since the routing table will be periodically recalculated from

scratch, implementation may require two route tables, one "in

progress" and one "in service".  When the route calculation is

complete, the "in progress" table becomes "in service" and the old one

is cleared for reuse.  This allows packet forwarding to continue while

the completed paths table is being converted into the route table.

A manual route table may also be maintained.  This should be overriden

by RSPF-generated entries.  Manual routes are useful defaults before

RSPF generates routing entries, for destinations not reported on by

RSPF, and for interworking with other routing protocols.

Appendix I.  Router parameters

Every router must set a number of parameters in order to properly

operate.  While RSPF builds its routing matrix automatically, overall

network efficiency and stability may require some fine-tuning based

upon experience.  These include parameters set for each data link

or subnetwork layer entity (i.e., each radio channel) and for the

router in general.

Link/subnet settings:

Set Link cost:  This is the cost parameter based upon the link speed

and type.  Since the overall cost of the end-to-end path is

minimized by the RSPF spanning tree, link cost should be set to

arrive at the best overall network performance.  The legal range

is 1-127.  This is sent in routing update bulletins.

Node settings:

Add/Delete Node group: [IPaddr]/bits {cost}.  This allows a node to

announce its ability to serve a group of nodes, identified using

the standard NET convention of address/significant bits.  Thus a

node group setting of [44.56.0.1]/25 will match all nodes from

[44.56.0.1] to [44.56.0.127].  Cost is optional; if set, this

cost to will be propogated to reach such nodes; otherwise, the

link's default is used.

Set horizon link:   This sets the horizon value for the node's

routing bulletins apropos 32-bit addresses of other connected

routers.  This should be high enough to propogate across the

backbone.

Set horizon group:  This sets the horizon value for the node's

routing bulletins apropos node group addresses (fewer than 32

bits matched).  Usually matches the horizon link value.

Set horizon local:  This sets the horizon vaue for the node's

routing bulletins apropos full link addresses (32 bits) **to

non-routers** within the router's node group area.  This is set to

a low value so that only other routers serving the same or overlapping

node group(s) will receive these messages.

Set horizon portable:  This sets the horizon value for the

node's routing bulletins apropos full link addresses (32 bits)

not within a node group.  This allows portable end nodes to

have their location in the network propogated farther than

the local horizon; this is usually set the same as horizon group.

Set broadcast timer:  This sets the time, in seconds, between

router-router hello (RRH) broadcasts.  Initial suggestion:  900.

Set suspect timer:  This sets the time, in seconds, after which

an adjacency is "suspect" if no packets are received from it.

An ICMP echo is then issued.  Initial suggestion:  900.

Set suspect count:  This sets the number of times an ICMP echo

(ping) should be sent after a node is suspect, before it is

removed from the adjacency list.