QCkt

The Quick Circuit Analyzer

Written by Kevin McClaning

Reference Manual

Version 1.00

January 1988

(c)  Copyright 1988 by RadioWare

All Rights Reserved

RadioWare Inc.

P.O.Box 2016

Columbia, Md   21045

This document describes the QCkt circuit analysis package.

This  version   of  QCkt  is  being  distributed  under  the

ShareWare concept.   The  ShareWare  concept  is  based  on three

principles:

1. People  need to try programs before they buy them to

see if they are useful.  You have to use  a program for

a while  before you can determine whether it's suitable

for what you want to do.

2. Software authors can be supported directly by users,

3. Copying and networking,  one of  the major strengths

of electronic data, can be encouraged.

If  user-supported  software  works, then everyone benefits.

The user will benefit by receiving  quality products  at low cost

along with  being able to try out a package before purchase.  The

author benefits by being  able to  enter the  commercial software

market without large amounts of capital.

User-supported  software  is  generally  not  public  domain

material: most programs carry a copyright  notice, including this

one.   The author  has licensed  you to  use and copy the program

under certain conditions.  Likewise, ShareWare is not intended to

be free  software.  It is intended to provide quality software at

a low price, while directly supporting the author.

The bottom line is this - If you're still using  a ShareWare

product after a few weeks, then the program is worth something to

you and you should make a contribution.

If you like this program  and  find  it  of  use,  then your

registration of $25 will be greatly appreciated.  It will be used

to support future versions  of the  program as  well as providing

encouragement to produce other programs.

What will you get for your registration?  First, I will send

you the next major revision at no extra charge.   After that, you

can  always  get  the  current  version  for  a  distribution and

handling  charge  of  $10.    Secondly,  your  registration  will

encourage me  to revise  and improve QCkt.  Thirdly, I'll put you

on my mailing list for revisions  of QCkt  and other  programs we

write.  Lastly, you'll get support.  I will always respond to the

queries of registered users  but I  may not  have time  to answer

questions from non-registered users.

All users of QCkt may :

1.  Copy  and  freely distribute QCkt as long

as:

a. No fee is charged, except for copying

and distribution  charges not to exceed

$12.

b. QCkt is distributed only in it's

original, unmodified  form with all of

it's documentation.

c. It is not distributed with or as any

part of  any other  package (software or

hardware), unless prior written  permission

has been granted by RadioWare.

2.  If  you  are  using  this  product  in  a

commercial  environment,  then  your  company

MUST  register  or  see  about getting a site

license.  That way, I'll stay in business and

you'll get support.

Send all inquires to :

RadioWare

P.O. Box 2016

Columbia, Md   21045

Make all  checks payable  to RadioWare, Inc.  Please use the

enclosed order form and please PRINT your full name and address.

Site licenses and commercial distribution licenses  are also

available.    I  will  do  customized versions at very reasonable

prices.

I would really like to hear your comments about QCkt.   Even

if you're  not a  registered user,  please drop  me a line if you

have any comments, criticisms or complaints.   With  such help, I

will modify QCkt to produce a better, easier-to-use program.

Occasionally, I  do a little teaching and I've found QCkt to

be very helpful in this  respect.    Concepts  such  as matching,

transmission  lines  and  filtering  become  very  clear when the

student can  twiddle  with  component  values  in  an interactive

environment (that's  where many  of the examples included on this

disk have come from).  The  students get  a good  feeling if they

see  the  filter  they  designed  out  of  a book perform like it

should.  Also, they can see the effects of finite component Q and

parasitic components  on circuit  performance.  Anyway, if anyone

out there  uses QCkt  in a  teaching environment,  I would really

like to  hear about  their experiences.  Personally, I think this

will be the wave of the future.

The Fine Print ...

Disclaimer

RadioWare makes no representation or warranties with respect

to  the  content  hereof  and  specifically disclaims any implied

warranties to the suitability of this program for  any particular

purpose.    You,  the  user,  must  determine  that yourself.  In

addition, you should understand that using a program of this type

on an IBM PC or similar compatible machine has inherent risks and

that you may inadvertently damage or destroy valuable programs or

data.   RadioWare expressly  declines to assume liability for any

use of  this  program  by  you  and  your  use  of  this programs

constitutes  your  agreement  to  hold  us  blameless.  RadioWare

reserves the right to make  changes  from  time  to  time  in the

context hereof without obligation to notify any person or persons

of such changes.

Trademarks

MS-DOS is a registered trademark of Microsoft Corp.

PC-DOS and IBM PC are registered trademarks of IBM Corp.

Turbo  Pascal  is   a   registered   trademark   of  Borland

International Inc.

The  Smith  Chart  is  registered  by  P.H.Smith  of  Analog

Instruments, Inc.

Table of Contents

1    ..............................  Introduction

2    ..............................  An Example

3    ..............................  Entering and Saving A

Circuit

4    ..............................  Entering Frequency

Sweeps

5    ..............................  Variable Components and

the Tune Mode

6    ..............................  Plotting the Frequency

Response

7    ..............................  The Smith Chart

8    ..............................  Some Examples

9    ..............................  How QCkt Works

10   ..............................  QCkt Limitations

Appendices

A    ..............................  Technical

Specifications

B    ..............................  Acknowledgements

C    ..............................  References - Credit

Where It's Due

D    ..............................  Component Summary

E    ..............................  Running Aspect.Com

F    ..............................  Order Form

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Chapter One : Introduction

This  manual  describes  the  operation  and use of QCkt, an

interactive circuit analysis program  for  the  IBM-PC  and close

compatibles.    This  manual  contains a description of the files

found  on  the  disk,  an  overview  of  program  features  and a

reference guide for all of QCkt's commands.

QCkt's  personality  is  geared  toward  the  practicing  RF

engineer.  I do a lot of RF circuit design and analysis in the 20

-  1000  MHz  range  and  none  of the reasonably priced programs

currently on the market did what I wanted  to do.   I  wrote QCkt

over many months with the following goals in mind:

1.  Fast  data  entry  -  allow  the  user to enter the

circuit of interest in a short time.

2. Fast sweep time -  to  allow  the  user  to  see the

effects of component changes almost immediately.

3. Quick  feedback when changing component values - one

of the problems I have with  programs currently  on the

market is  that they don't let you twiddle with circuit

components and immediately see the result.

4. Quick access to the circuit from almost any point in

the  program  -  you  can  change  almost  any plotting

parameter or circuit value while  in  one  of  the plot

modes  and  immediately  see  the  results  on  circuit

performance.

QCkt will  run on  an IBM-PC  or close  compatible.  Minimum

system requirements  are 256-K of memory, PC-DOS or MS-DOS 2.0 or

greater and at  least  one  floppy  disk  drive.    QCkt  needs a

color/graphics card  with either a color or monochrome monitor to

function properly.

You will need an 8087 numeric co-processor chip installed in

1

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

your machine  to run  QCkt-87, the 8087 version of QCkt.  QCkt-87

runs about 4 times faster than  QCkt  --  a  big  difference when

evaluating large circuits.

Due to  the Turbo  Pascal compiler used to generate QCkt and

QCkt-87, their file structures are incompatible.   That  is, QCkt

cannot  read  or  write  QCkt-87 files.  Likewise, QCkt-87 cannot

read or write QCkt files.

The files included with this package are :

QCkt.Com - This contains the main  driving routines  used by

QCkt.

FarCode.Com  -   This  module  contains  the  code  for  the

rectangular  and  Smith  Chart  plotting  routines.    QCkt  will

terminate  with  an  error  message  if  this  file is not on the

default drive at start up.

Aspect.Com - This program generates the aspect ratio of your

monitor.    If  the  Smith  Chart  isn't round on your particular

monitor and you can't correct it  by fiddling  with the controls,

run Aspect.Com  to correct the problem.  Note: Aspect.Com creates

a short file called Aspect.Dta on the default  drive.   This file

contains the  aspect ratio  of your  monitor which  QCkt reads on

start up.

Aspect.Dta - This file contains  the  aspect  ratio  of your

monitor,  if  needed.    See  the entry under Aspect.Com for more

information.

Ex*.Ckt - These are example circuit  files discussed  in the

manual (Read on).

QCkt.Doc - This manual in ASCII format.

In order to run QCkt, you must have QCkt.Com and FarCode.Com

on the default drive and directory.  If you need  Aspect.Dta (see

Appendix E  for more information), it must also be on the default

drive and directory.

One final note before we get into it.  If you're a new user,

go through  the example in Chapter 2 before you do anything else.

This will help acquaint you with some of  the features  and power

of QCkt.   Then,  skim the  rest of  the manual  while sitting in

front of your computer.  Try things out.  Finally, go through the

examples given in Chapter 8.   This procedure will get you up and

running quickly and painlessly.    Use  the  other  parts  of the

manual for reference or when you're having trouble.

2

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

3

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Chapter Two - An Example

If  you're  like  me,  then  you don't want to read too much

before you fire the computer up to play.   This  chapter will run

through  an   example  which  will  demonstrate  most  of  QCkt's

capabilities.

The first example circuit  we'll go  through is  on the disk

under the name Exbpf214.Ckt.  As the name implies, this is a 21.4

MHz bandpass filter.  It is a Chebychev filter with a 1  Mhz 5-dB

pass band, capacitively matched to 50 ohms.

Before we  get to  the example,  let's clear up a few terms.

When I ask you to hit the Alt-S key, for example, this means hold

down the  Alt key  (lower left hand corner of your keyboard) and,

while still holding the Alt key down, strike the S  key (either a

capital-S or small-S will do - it makes no difference).

Another item  to clear up is the subject of default answers.

Typically, when the program asks the user for input, it will show

a default  value in parenthesis.  The value of the default varies

depending upon how  you  answered  the  question  last  time, the

present state  of the circuit currently in memory and what you're

doing at the time.  Anyway, to accept  the default,  all you have

to do  is hit  the return  key.  For example, suppose the program

asks:

"Which Component (A, B, C, D or E) : (A) ...".

The choices you are allowed to  enter are  A, B,  C, D  or E

(either  upper  or  lower  case)  and  answer  A  is the default.

Hitting the return key is the same as typing an "A", then hitting

the return key.

Finally,  when  the  circuit  lists  possible responses to a

question, you can respond  with a  number.   Consider our example

from before - The program asks:

"Which Component (A, B, C, D or E) : (A) ... "

The choices you can enter are A, B, C, D or E.  You can also

4

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

enter 1 for A, 2 for B, 3 for C, etc. up to 5 for E.  In general,

you can enter a 1 for the first answer listed, a 2 for the second

and so on.  This feature  speeds things  up slightly  by allowing

you to keep your right hand over the numeric keypad at all times.

Consider another example - suppose the program asks:

"Continue (Y or N) : (Y) ... "

Hit return for Yes.  You can hit either "n", "N" or "2" for No.

From the DOS prompt and with  the QCkt  disk in  the default

drive, type  "QCkt" and  wait for  the main  menu.  Then, get the

Exbpf214.Ckt file from the disk by selecting item 11.   This puts

up  the  QCkt  retrieve  file  menu.    Select  option 2 and type

"Exbpf214" (the program automatically  appends the  ".ckt" if you

don't enter  an extension).   This  will load the 21.4 MHz filter

file from the disk and return you to the main menu.

Loading a file this  way loads  a description  of the actual

circuit to be analyzed as well as several other system parameters

such as component Q, 2 different frequency sweep ranges, plotting

parameters and a host of other items.

At the  QCkt main  menu, select  item 3 to plot the gain and

return loss of the filter we just retrieved from the  disk.  This

brings  up  two  more  menus  - don't bother with them right now.

Just hit the return key twice to accept  the default  options and

get a gain plot of the filter.

The gain plot is really two different plots displayed on the

same scale.  The solid line you see  is the  transfer function of

circuit  in  memory.    This  is the power dissipated in the load

resistor divided by the maximum power available from  the source.

The second  plot made  up the  little square  boxes is the return

loss of the circuit.  This function reflects how well the circuit

is  matched  to  the  system characteristic impedance - the lower

this number is, the closer the circuit input impedance is  to the

system characteristic impedance.

In the  upper left  corner is  the current cursor frequency.

This is the frequency currently being evaluated and it relates to

the  small,  hollow  box  currently  at the left hand side of the

graph (it's about 35 dB down).  If you press the '+'  or '-' keys

on the  numeric keypad,  this evaluation frequency will increment

or decrement accordingly.  Go ahead and try it now.

You'll notice that as  you change  the evaluation frequency,

the  numbers  at  the  bottom  labelled Gain, Phase, R.L. (return

5

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

loss) and G.D. (group  delay) also  change.   These number always

reflect  the  characteristics  of  the  circuit  in memory at the

evaluation frequency.

Another way to change the evaluation  frequency is  to press

the Alt-F  key.   A window  opens up  and asks you to enter a new

frequency.  Enter 21.4 MHz for the new  frequency.   When you hit

return, the  previous screen  is restored  and updated to reflect

what the circuit is doing at 21.4 MHz.  The frequency cursor (the

small hollow box on the graph) is also moved to 21.4 MHz.

Now, hit  the Alt-I  key.   This will  bring up  a help menu

which contains all of the  commands  you  can  use  while  in the

plotting mode.   It  might be  a good idea to do a Shift-PrtSc at

this point to get a hard copy.  Strike any key to get back to the

plotting screen.

Try  the  Alt-W  key.    This  key lets you twiddle with the

frequency sweep and plot  parameters.    The  menu  that  you see

displays the  two sweeps currently in memory (labelled #1 and #2)

along with their current step sizes.  Take note that  the current

sweep is #1 by looking at menu option 4.  Change the active sweep

from sweep #1 to sweep #2 by selecting option 4.  When the screen

repaints, you'll see the active sweep has now become #2.  Hit the

return key once more to return to the plot.

When we return to the plot,  you'll notice  that nothing has

changed.   QCkt simply  saves the current graphics screen when it

calls one of the Alt functions and restores it upon completion of

the Alt  task.   To re-sweep  the circuit with the current active

sweep (now sweep #2), press Alt-S.  The screen will  be repainted

with the new active sweep.

This entire  operation of striking Alt-W to edit the sweeps,

changing the active sweep  to #2  with one  of the  menu options,

returning to  the plotting  screen and doing an Alt-S to re-sweep

the circuit is done often.  The whole process can be accomplished

by hitting the Alt-A key to toggle the active sweep.  Try it now.

The active sweep will  be changed  back to  sweep #1,  the screen

will be  repainted and  the circuit will be re-swept according to

the first sweep's parameters.

Let's try one more thing before we leave  this plot.   Press

the Alt-G  key.  This key allows the user to enter global circuit

and plotting parameters.   These  parameters  are:  the  load and

source  resistance,  the  system  characteristic  impedance,  the

frequency where the electrical  length  of  T-Lines  is specified

and, finally, capacitor and inductor quality factors or Q's.

After  hitting  Alt-G,  the  program will ask you a question

6

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

about system characteristic impedance.  Hit return to  accept the

default value  of 50  ohms.   Likewise, just hit return to accept

the defaults for load  and source  resistance and  for the T-Line

question.  When the program asks for inductor Q, take note of the

default value.  This number, along  with capacitor  Q, was loaded

from disk  when we  loaded the circuit.  The value displayed here

(inductor and capacitor Q both = 10000) is  artificially high and

doesn't reflect  the realizable,  lossy components we have to use

in the real world.    Change  the  inductor  Q  value  to  a more

reasonable  value  of  250  and  the capacitor Q value to 1000 by

entering these numbers in  response to  the appropriate questions

and hitting return.

Upon  answering  the  capacitor  Q question, you should find

yourself  looking  at  the  gain  plot  of  the  circuit  you saw

previously.    Make  a  mental  note of the filter insertion loss

(about 0) and the depth of  the ripples  (about 5-dB).   Now, hit

Alt-S to  sweep the circuit with the new, more realistic Q values

that we just entered.  You'll note that  the "real-world" circuit

has an increased insertion loss (about 3 dB) and the ripple depth

has decreased (also to about 3 dB).   One of  the great strengths

of QCkt  is that  it allows  you to twiddle with component values

and quickly see the results on a plot.

Finally, hit the Alt-X key to exit the gain plotting routine

and return to the main menu.

Main  menu  options  4  and  5  (plot phase and group delay,

respectively) perform much the same as the gain plotting routine.

Try them out for a spin!  Remember that on-line help is available

through the Alt-I key.

When you're done playing  with  the  phase  and  group delay

options,  return  to  the  main  menu  (via Alt-X) and reload the

original circuit by entering option 11 and retrieving the default

file  (which  is  now  Exbpf214.Ckt).    This is just in case you

accidentally changed anything important  while you  were fiddling

around.   Now enter  option 6,  the Smith Chart plotting routine,

from the main menu.   Hit  return  twice  to  accept  the default

options  and  you'll  be  dropped  into  the Smith Chart plotting

routine.

This option  lets  you  plot  the  input  impedance  of your

circuit on the Smith Chart.  Hit Alt-F and enter 21.4 to move the

frequency cursor to 21.4 MHz.   You'll  see  that  at  our center

design frequency of 21.4 MHz, things don't look so good.  Looking

in the upper left-hand corner of the screen,  we can  see that we

have a  VSWR of  about 10, our input impedance is nowhere near 50

Ohms and our return loss is about 0 dB (indicating  a pretty poor

match).

7

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Since  the  center  of  the Smith Chart represents a perfect

match, we are interested in  those  frequencies  where  our input

impedance  is  near  the  Smith  Chart  center.    Increment  and

decrement the frequency cursor using the '+' and '-' keys  by the

numeric keypad  to find  the two  frequencies closest to 50 Ohms.

You should get about 21.05 and  21.80 MHz.   If  you don't, Alt-F

your way to these frequencies to verify that they are indeed near

the center of the chart.

You'll notice that the impedance  plot  on  the  chart looks

fairly  rough.    In  the  part  of  the spectrum where the input

impedance  of  the  circuit  is  changing  rapidly,  the plotting

resolution is  pretty coarse.   You can see the points calculated

by the program connected with straight lines.

One of the options we didn't explore while we were doing the

gain plot is the Alt-H key.  This key will halve the current step

size, giving finer plotting  resolution but  increasing the sweep

time.  Press Alt-H twice to plot 4 times as many points, then hit

Alt-S to sweep the circuit with  the new  step size.   You'll see

that the plot is a lot smoother, but that the sweep time has also

increased.  You can double the current step size using  the Alt-D

key.

If you're  not all  ready there, Alt-F your way back to 21.4

MHz and make a mental note of the VSWR at that point.   Then, hit

the Alt-C  key to  draw circles  on the  Smith Chart.   The Smith

Chart Circle menu will come up, asking you  which type  of circle

you want to draw on the chart.  Just hit the return key to accept

the default of constant VSWR circles.  Enter the VSWR at 21.4 MHz

and hit  return.   When the screen repaints, you'll see the extra

VSWR circle.  It will be centered about  the center  of the chart

and should pass through the frequency cursor at 21.4 MHz.

And again,  just for the heck of it, enter Alt-G to edit the

global circuit variables again.   As before,  change the inductor

Q  to  250  and  the  capacitor  Q to 1000.  When the Smith Chart

screen returns, hit Alt-S to sweep the circuit again and  see the

effect of realistic component Q.  It does make a difference.

8

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Chapter Three - Entering and Saving A Circuit

Entering a circuit into QCkt is really quite easy.  Remember

that, at present,  QCkt  can  handle  only  ladder  circuits with

resistive terminations.   For those of you who don't know, ladder

circuits are circuits which contain only shunt and series element

connected in  a cascade  fashion.  This particular topology turns

out to be quick and fairly easy to analyze.

Before entering a  circuit,  it's  a  good  idea  to  have a

drawing of  the circuit  in front  of you.   Label the components

starting with #1 at the load end and continuing on  to the source

resistor.  Don't number the source or load resistors themselves.

From  the  QCkt  main  menu,  select  option  1 to clear the

circuit and enter a new one.  The program will ask you  if you're

sure you  want to  do that - answer yes by typing a "y" or a "1".

Any other  response will  send you  back to  the main  menu.  The

program  will  then  ask  you  how  you  want  the global circuit

variables set up.  Enter values for the system impedance  and the

load and source resistance (all are typically 50 or 75 Ohms).

The next  question deals  with transmission lines.  When you

enter a transmission line,  you'll have  to tell  the program the

desired characteristic  impedance of  the line  you want to enter

and it's electrical length in degrees  at a  specified frequency.

This is  where you enter that frequency.  It will be the same for

all of the transmission lines in your circuit.

The last  two  questions  ask  for  the  quality  factors or

Q-values for  inductors and capacitors.  The defaults are 250 and

1000 for inductors and capacitors, respectively, which are pretty

representative of realizable high-quality components.

Hitting return  after the  capacitor-Q question  leads us to

the component selection menu.  The components the user is able to

include in his circuits are:

1.  Capacitors  -  The  value of all capacitors in this

program  are  currently  given  in  picofarads.     All

capacitors have  the same  global Q value which is held

constant  with  frequency.    The  default   value  for

capacitor Q is 1000.

9

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

2.  Inductors  -  The  value  of  all inductors in this

programs are currently  given  in  microhenries.   Like

capacitors, all  inductors have the same global Q value

which can be (and usually is) different from the global

capacitor Q.  The default value for inductor Q is 250.

3. Resistors  - The values of all resistors are entered

in Ohms.

4. Parallel  LC  -  An  inductor  and  capacitor hooked

together in  parallel.   As before,  the capacitance is

given  in  picofarads  while   the  inductance   is  in

microhenries.   Global capacitor  and inductor Q losses

are applied to this combination.

5. Series LC -  As in  #4, but  the two  components are

connected in series.

6. Parallel  RC - A resistor and capacitor connected in

parallel.

7. Series RC -  A resistor  and capacitor  connected in

series.

8. Parallel  RL -  A resistor and inductor connected in

parallel

9. Series RL - A resistor and inductor in series.

10. T-Line - This is a lossless transmission line.  The

user enters a unique characteristic impedance (in Ohms)

and electrical length (in degrees).  If the  line is in

shunt with  the circuit,  the user is asked if the line

is open- or short-circuited.  The  electrical length is

the number of degrees the line appears to be at a given

frequency.  The  frequency  is  specified  as  a global

value and it is the same for all of the T-Lines entered

in this program.  The default is 100 MHz.

11. Parallel Resonator - This is  simply a  parallel LC

circuit (see  item #4  above) specified  in a different

way.  Instead of  asking for  a value  of L  and C, the

program asks  for a  value of L (in microhenries) and a

center frequency (in MHz).  The  parallel LC  will then

be  made  to  resonate  at the given frequency.  Global

capacitor  and  inductor  Q's   are  applied   to  this

component.

12. Series  Resonator -  The same  component as in item

#11,  except  that  the  inductor  and   capacitor  are

10

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

connected in series.

13. Defined Impedance Plane - This is an operation more

that it is a component.  If one of these  is present in

the circuit, the Smith Chart and return loss plots will

reflect the impedance of  the circuit  looking from the

impedance plane  toward the  load.  The Smith Chart and

return loss plots will not reflect  the input impedance

as the source sees the circuit.

If  there  is  more  than  one  of  these  defined in a

circuit, the last impedance  plane (the  one closest  to the

source) is used.

14.  Duplicate  an  Existing  Part  -  This  is  a very

convenient  operation  which   allows   any  previously

defined  part  (ie.  a  part  with  lower number) to be

duplicated.  This is useful for  some types  of filters

and matching  circuits which  exhibit a  high degree of

symmetry.

A little sidenote - In all cases, you  are allowed  to enter

negative values  for components.   Mathematically,  this is not a

problem for the evaluation software, but interpretation is  up to

you.    Doing  this,  however,  can mess up the graphics plotting

routines.  They won't crash, but will usually try to  plot off of

the outlined grid.

I allow  negative components  for the  simple reason that it

can be interesting to plot such  things.   You can  also settle a

lot of silly arguments about circuit theory when they come up.

From this  point, enter the components in order; starting at

the load end of  the circuit  and proceeding  towards the source.

The program  will prompt  you for  the information it needs about

any particular part.  When you're finished, enter  99 when you're

asked for  a component  (or just hit return).  This will take you

back to the main menu.

At the  main menu,  enter option  #2 to  take a  look at the

circuit you  just entered.  The top three lines of the new screen

give information on the  global  circuit  parameters  you entered

when you started a new circuit.

After  that,  the  program  displays  the current circuit in

memory starting at the load with  block #1  and proceeding toward

the source with increasing block numbers.  After the block number

is the component type (inductor, capacitor, etc.), followed  by a

description of  the way the component is connected in the circuit

(either shunt  or  series).    Finally,  the  component  value is

displayed along with the proper units (uH, pF or Ohm).

11

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

If you  want to  change a  particular circuit element, enter

the block number of  the component  you want  to change.   You'll

find yourself back at the QCkt component menu where you can enter

a new component or  change the  value of  the present  one.  When

you're  done,  QCkt  takes  you  back  to the circuit description

screen.

A convenient feature of QCkt is that it allows you  to enter

new components from the circuit description screen.  For example,

if the maximum component  number in  your circuit  is 9,  you can

enter 10  for the  number of the block you'd like to change.  You

can then add a new component to your circuit.

If you like, you can also  change the  global variables from

this screen by entering 99 as the component you'd like to change.

QCkt will then take you to the global editing screen.

Main menu options 10 and 11 allow you  to save  your circuit

file on disk.

Select  option  11  to  retrieve  a  circuit  from the disk.

You'll come to the QCkt file menu which is reproduced here:

Get Disk Menu - QCkt (Version X.XX)

-----------------------------------------------------------------

Pick an Option:                    Default File is : Work.Ckt

1.  Get Circuit From the Default File

2.  Get Circuit From Another File

99. Return

-----------------------------------------------------------------

What'll It Be : (1) ...

You can get a file from the default  circuit file (Work.Ckt,

here) or  from any other file.   If you retrieve or save anything

in any other file, that file then becomes the default  file.  For

example, when  we retrieved the example circuit Exbpf214.Ckt from

the disk, that became the default file.   This makes  updating of

your circuit  files as  you're working  on them  pretty easy.  To

save something in the  default file  amounts to  selecting option

#11 from the QCkt main menu and hitting return twice.

If you  need to  save your  circuit in a file other than the

default file, select option #2.   QCkt will  ask for  the name of

the  file  you  want  to  store  data in.  Only twelve characters

(8-character name, decimal point and a 3-character extension) are

allowed.   The circuit  will be saved on the default drive in the

current directory.

12

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

If you  enter a  file name  without an  extension, QCkt will

tack ".Ckt" onto the end of the name you enter.  If you specify a

file that doesn't exist,  the program  will beep,  tell you about

your mistake and take you back to the main menu.  No data will be

changed.

QCkt main  menu option  10 (the  save to  disk option) works

exactly the same way as the retrieve function does.

The following  is a  list of  the information QCkt stores on

disk:

- Total number of blocks in your circuit,

- The primary, secondary and tertiary items of each element

(the Z0, length and Short/Open Circuit status of

transmission lines,  for example).   Also, whether each

element is in series or shunt,

- Global Inductor and Capacitor Q,

- The source and load resistances,

- The frequency at which transmission line electrical

are valid,

- The system characteristic impedance,

- The number of defined parallel branches,

- The following data for each sweep:

- Start frequency

- Stop frequency

- Increment frequency

- The  value of the bottom of the

linear gain plot

- The value of  the top  of the

group delay plot

- The  tic mark interval for the

horizontal and  vertical  axes

for the  gain, phase  and group

delay plots

- The following data for the variable components:

- The block number  of the variable

component

- Whether the variable component is

the primary, secondary or

tertiary element of the block

13

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Chapter Four - Entering Frequency Sweeps

Before you can plot your circuit, you have to set up several

parameters to tell QCkt how to plot your data.   These parameters

include  start  and  stop  frequencies, vertical ranges, tic mark

spacings and the like.  You can do this from  the QCkt  main menu

option #7 (among other places in the program).

From the  QCkt main menu, enter option #7 to get to the edit

sweep menu.  This  menu displays  the current  parameters for two

separate frequency  sweeps, labelled  sweep #1 and sweep #2.  The

menu displays the start, stop and step frequencies.

The circuit is evaluated from  the  start  frequency  to the

stop frequency, incrementing by the step frequency.  For example,

if the start frequency is 100 MHz, the stop frequency  is 200 MHz

and the  step frequency  is 1  MHz, the circuit will be evaluated

and plotted at 100 MHz, 101 MHz, 102 MHz ... 199 MHz  and 200 MHz

for a  total of  101 points.  The more points you pick, the finer

the graph will be but the  longer  it  will  take  to  complete a

sweep.

Some  sidenotes   -  Anytime  0  MHz  is  encountered  while

analyzing a circuit, the circuit will be evaluated at 1 Hz.  This

allows  you  to  enter  0  MHz  as  a start or stop frequency and

prevents divide-by-zero run time errors.

You can  enter  negative  frequencies  without  any problem.

Realistically,  this  is  pretty  useless although mathematically

interesting.

If you enter a frequency  step  size  of  0  MHz,  QCkt will

default the step size to the following expression:

Step Size = Abs(Stop Freq - Start Freq) / 100

which will produce 101 evaluation points.

The edit  sweep menu  presents you  with four  options.  The

first three  allow you  edit either  the first  sweep, the second

sweep or both sweeps.  We'll get to that in a minute.

14

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

The  fourth  option  allows  you  to toggle the active sweep

between #1 and #2.  The active sweep is the  sweep used  when you

plot anything.   The  inactive sweep  is always there, waiting to

become the active sweep whenever you want.

Two sweeps allow you to  look  at  a  circuit  in  two ways.

First,  you   can  look  at  the  circuit  up  close,  where  the

interesting things are happening (in the  passband of  a bandpass

filter, for  example).   Secondly, you can get an overall picture

of the circuit as it behaves over a large frequency range (to get

an ideal of ultimate attenuation and that kind of thing).

Anyway, select  option #1  from the  QCkt edit sweep menu to

edit sweep #1.  QCkt will remind you  which sweep  you're editing

at the  top of the page, then ask you for the new start, stop and

step frequencies.  Either enter new frequencies or  hit return to

accept the  default values  in parentheses.   For example, if the

screen looked like:

Edit Sweep #1

-------------------------------------------------------------

Start Freq (MHz) : (19.4000) ... 15

Stop Freq (MHz) : (23.4000) ... 28

Step Freq (MHz) : (0.0500) ...

-------------------------------------------------------------

It means that the previous start, stop and  step frequencies were

19.4 MHz,  23.4 MHz and 0.05 MHz.  The user entered new start and

stop frequencies of 15 and 28 MHz, but accepted the  default step

frequency of 0.05 MHz.

After you've entered the step frequency, QCkt will ask for a

set of plotting guidelines.  Since  QCkt deals  only with passive

devices which  have no  gain, it  reminds you that the top of the

X-Y grid will be 0 dB, then asks for the attenuation value at the

bottom  of  the  screen.    Again,  just hit return to accept the

default value in parenthesis or enter a new value.

Now, QCkt  will  ask  for  the  tic  mark  interval  for the

vertical grid.   This  tells the  plotting routine  how often you

want to draw a horizontal line across the X-Y grid.

Finally, QCkt will ask  how often  you want  a vertical line

drawn  to  mark  off  frequency.    It  then asks for the maximum

expected group delay over  the  frequency  range  and  finally, a

group delay tic mark interval.

15

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

To  recap,  if  the  screen  looked  like this when you were

finished:

Edit Sweep #1

-------------------------------------------------------------

Start Freq (MHz) : (19.4000) ... 15

Stop Freq (MHz) : (23.4000) ... 28

Step Freq (MHz) : (0.0500) ...

-------------------------------------------------------------

Vertical Parameters

The Top of the Grid is 0 dB.

How Far Down is the Bottom (dB) : (40.0) ... 50

Enter the Vert Grid Tic Mark Interval (MHz) : (5.0) ...

-------------------------------------------------------------

Horizontal Parameters

Enter the Horiz Grid Tic Mark Interval (MHz) : (2.0) ... 1

-------------------------------------------------------------

Group Delay

Enter the Maximum Expected Group Delay (ns) : (1200) ... 1000

Enter the Group Delay Tic Mark Interval (ns) : (200) ...

-------------------------------------------------------------

It means that the X-Y grid for this sweep will be plotted from 15

to 28  MHz in 0.05 MHz steps.  The vertical part of the grid will

go from 0 dB at the top of the screen  to -50  dB at  the bottom.

QCkt  will  partition  the  vertical scale into 5 dB/div sections

(ie.  it  will  draw  a  horizontal  line  every  5  dB)  and the

horizontal scale  into 1 MHz divisions (ie. a vertical line every

1 MHz).  When plotting group delay, the vertical scale will cover

0 to 1000 ns and will be marked off every 200 ns.

16

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Chapter Five - Variable Components and the Tune Mode

The  concept  of  variable  components  is  one  of the most

powerful features of QCkt.  As  I said  in the  beginning of this

manual,  I  wanted  to  be  able  to  change component values and

immediately see  the effect  on both  the transfer characteristic

and input  impedance of  the circuit.  This operation is realized

through the tune mode using variable components.

Basically, you can select up to five components  (labelled A

through E)  that you  want to vary.  Then, when you are at any of

the plotting screens  (gain,  phase,  group  delay  or  the Smith

Chart), you  can change  the value  of the  selected component by

entering the Tune Mode.

Before we begin, load  the circuit  called Exbpf214.Ckt from

disk.  We will use this as the example here.

To  assign  circuit  element  as  variable components, first

enter your circuit into the machine (either by  hand or  from the

disk drive),  then select  the QCkt  main menu  option 8 labelled

"Edit  -  Variable  Components".    This  will  take  you  to the

following menu:

17

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Edit Variable Components - QCkt (Version X.XX)

-----------------------------------------------------------------

Current Variable Components are :

Component A :

3    Inductor    Shunt    L = 1.0000 uH

Component B :

4    Capacitor   Shunt    C = 34.430 pF

Component C :

5    Capacitor   Series   C = 2.0100 pF

Component D :

Undefined

Component E :

Undefined

----------------------------------------------------------------

What'll It Be ...

1.   Edit One Component

2.   Edit All Components

3.   View the Parts List

99.   Return

----------------------------------------------------------------

What'll It Be : (99) ...

First of all, this screen shows you the current state of the

variable components.    In  this  case,  component  A  is circuit

element #3.   That  component is  a 1.0  uH inductor connected in

shunt.  Similarly, component B is a shunt 34.43 pF capacitor.  It

is  block  #4.    Variable  component  C  is  a  series connected

capacitor (C = 2.01 pF).  Components D and E are not defined.

The menu options  allow  you  to  either  edit  the variable

component list (options 1 and 2) or view the circuit currently in

memory (option 3).

Select item #1 to edit  a  single  component  and  you'll be

asked which  component you want to edit.  Enter an "A" and you'll

be presented with the following menu:

Edit Component A

-----------------------------------------------------------------

Present Part Is:

3    Inductor   Shunt   L = 1.0000 uH

-----------------------------------------------------------------

Maximum Part Number is   9

Enter New Part Number for Component A : ( 3) ...

18

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Enter  a  new  part  number  if  you  want  to  change which

component in  your circuit  will be labelled as component A.  Hit

return if you want to accept the default.  For this example, type

an 8 to tell QCkt that you want component A to represent whatever

circuit element is in  block #8.   When  you hit  the return key,

you'll get the following menu:

Edit Component A

-----------------------------------------------------------------

Present Part Is:

3    Inductor   Shunt   L = 1.0000 uH

-----------------------------------------------------------------

Maximum Part Number is   9

Enter New Part Number for Component A : ( 3) ... 8

-----------------------------------------------------------------

New Part is:

8    Capacitor   Series  C = 20.0000 pF

-----------------------------------------------------------------

Present Value is:   20.000

Enter New Value ...

If you  want to  change the  present value of the component,

you can do so at this point.  It will be recorded as part of your

circuit and the old value will be lost, so take care.

For this example, change the value of the component to 10 by

typing "10".  Right before you hit return, your screen  will look

like:

Edit Component A

-----------------------------------------------------------------

Present Part Is:

3    Inductor   Shunt   L = 1.0000 uH

-----------------------------------------------------------------

Maximum Part Number is   9

Enter New Part Number for Component A : ( 3) ... 8

-----------------------------------------------------------------

New Part is:

8    Capacitor   Series  C = 20.0000 pF

-----------------------------------------------------------------

Present Value is:   20.0000

Enter New Value ... 10

QCkt will  not allow  you to assign certain components to be

variable.    The  following  components  wouldn't  make  sense as

variable components:

- The impedance plane (component #13)

-  The  duplicate  component  (component  #14)  although the

component being duplicated can be made variable.

19

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Hit return to get back to the Edit  Variable Component menu.

Hit return a second time to get back to the QCkt main menu.  Just

in case you messed something up,  retrieve Exbpf214.Ckt  from the

disk again  via main  menu option  11.  If you don't have a print

out of this circuit in front of you, this might be a good time to

make one  (use main  menu option  2 and the Shift-PrtSc key).  It

would also help to draw this circuit out before you begin.

Now, enter option 3 and hit return twice to get  to the gain

plotting  screen  and  you'll  see the now-familiar plots of this

circuit's gain and return loss  curves.    Just  to  refresh your

memory, this  is a  bandpass filter  with a  1 MHz, 5 dB passband

ripple.

Using the variable components we've set  up, we're  going to

try to  narrow up  the bandwidth  of this filter and decrease the

center frequency insertion loss.  Alt-F your way to 21.4 MHz, the

center frequency  of this  filter, and note the insertion loss of

around 5 dB.

Before going any further, we need to talk about the variable

components of circuit we're about to adjust.  The bandpass filter

is made up to two identical,  parallel LC  circuits (components 3

and 4.   Components  6 and 7 are duplicates of 3 and 4).  The two

tanks are coupled by  a  single  capacitor  (component  5).   The

resonant frequency  of the  tanks determines the center frequency

of the filter while the coupling  capacitor determines  the shape

and insertion loss of the passband.

Now, enter  Tune mode  by hitting  the Alt-T key.  Tune mode

allows you to change any of the variable  components at  will and

immediately see  the results  of the  change.  When in tune mode,

the last line of the screen prompts you to enter a new  value for

one of  the variable  components.   If you enter a number and hit

return, the circuit will  be  re-swept  with  this  new component

value in  place.  If you hit the Tab key, you will advance to the

next  variable  component  (ie.   if  you're   currently  editing

component A and hit the Tab key, the last line of the screen will

now ask you to enter a new value for component  B).   Hitting the

Escape key will take you out of Tune Mode.

Tab your  way to  component C  and change  it a few times by

entering a new value and hitting  return.   Note that  every time

you  change  a  component,  the  circuit will be re-swept without

showing the return loss plot.   The parameters  of the  filter at

the cursor frequency are also updated and displayed on the bottom

of the screen.

Adjust component C to get the  sharpest bandpass  shape with

20

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

the least  insertion loss.   The  center frequency will shift but

we'll compensate for that  in a  minute.    What  you're doing is

adjusting the  coupling capacitor  between two tanks.  Decreasing

this capacitor decreases the coupling between the two parallel LC

elements and hence tightens up the bandwidth.  You should be able

to get the insertion loss to less than 0.2 dB when component C is

1.11 pF.

You'll notice  that the  center frequency has shifted upward

to 21.6 MHz, the ripple has decreased significantly  and the 3-dB

bandwidth is now about 500 kHz.

With the cursor still at 21.4 MHz, start adjusting component

B.   Using  the  duplicate  component  function,  you're actually

adjusting the  two capacitors  in the  tank circuits.  We want to

lower the center frequency back to  21.4 MHz.   Play  around with

these components  until the frequency is roughly centered at 21.4

MHz.

If you're having trouble, the circuit I came up with had the

following  values:    Component  B  =  Component D = 35.43 pF and

Component C = 0.710 pF.  The final circuit had  an insertion loss

of about 0.16 dB with a 300 kHz bandwidth.

Just for  fun, Alt-X  your way  to the  QCkt main menu, then

enter option #6 to get to the Smith  Chart.   It's interesting to

see what varying the coupling capacitor (component C) does to the

input impedance on the Smith Chart.  Enter the tune  mode (Alt-T)

and  adjust  Component  C.    As  before,  each time you adjust a

variable component you'll get another sweep.

21

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Chapter Six - Plotting the Frequency Response

Now on to the fun part  - plotting  things.   QCkt will plot

the following items on an X-Y grid:

1. Gain  and return  loss -  These are  measures of how

well  the  circuit  in  memory  accepts  energy  from a

resistive source (ie. return loss) and how efficient it

is at  transferring  that  energy  to  a  load resistor

(gain).  This is main menu option #3.

2. Phase  - This  is the phase of the voltage impressed

upon the  load  resistor  with  respect  to  the source

voltage.  This is main menu option #4.

3.  Group   Delay  -  Different  frequencies  can  take

different amounts of  time  to  get  through  a filter.

Group Delay is a measure of how long it takes an signal

placed upon the  input  to  produce  an  output signal.

Mathematically, group delay is calculated by taking the

derivative of the phase plot with respect to frequency.

For   more   information,   see  Chapter  9  on  QCkt's

Limitations.

All of the plotting  functions  work  pretty  much  the same

except for  what they  plot.   The information  displayed and the

place it's displayed  on  the  screen  doesn't  vary  nor  do the

Alt-Key commands.   I  tried to  get things as uniform in command

structure as possible.

From the QCkt main menu, enter option #3 for the gain/return

loss plot.   Actually, you could have entered either #3, #4 or #5

for the gain\return loss, phase or  group delay  plots, but going

through  the  gain\return  loss  plot  will  keep  this narrative

consistent with what you see on the screen.

The first menu that pops up asks whether  you want  to sweep

the circuit  currently in memory, edit the variable components or

edit the frequency sweeps  currently  stored  in  memory.   We've

already  covered  editing  the  frequency sweeps in chapter 4 and

we'll get to the  variable components  in a  later chapter.   For

22

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

now, just  hit return  to accept the default task of sweeping the

circuit in memory.

Upon hitting return, QCkt then asks if you want to  keep the

current  active  sweep  or  switch  to  the  other.    Enter  the

appropriate answer and hit return to get to the plot.

One thing to note: in the vast majority  of cases, answering

these questions usually boils down to hitting return twice before

you get to the plotting screen.  So  even though  these questions

seem cumbersome  now, they  really aren't much of a bother at all

once you begin using the program.

You are now at the QCkt plotting screen  and should  see two

distinct plots on the screen.  The solid line is the circuit gain

plot while the dotted line is the circuit return loss.

The top two lines  remind you  of what  you're doing.   They

tell you how to get to the help screen, reminding you of the file

you're playing with and  give you  the current  cursor frequency.

The second  line from  the bottom  of the  screen gives the gain,

phase, return  loss (labelled  "R.L.") and  group delay (labelled

"G.D.") at the cursor frequency.

The vertical  axis is  labelled along  the left  side of the

grid.  This  axis  is  labelled  in  dB,  degrees  or nanoseconds

depending upon  the plot  you're doing.   The  horizontal grid is

labelled at the bottom with the start and stop frequencies.

The  help   screen  is   displayed  by   typing  Alt-I  (for

information).  This information menu is reproduced below:

23

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Plotting Help - QCkt (Version X.XX)

-----------------------------------------------------------------

+/-   : Increment/Decrement Freq

Alt-A : Active Sweep Toggle

Alt-D : Double Freq Step Size

Alt-F : Enter New Cursor Frequency

Alt-G : Edit Globals

Alt-H : Halve the Freq Step Size

Alt-I : This Help Screen

Alt-N : Draw New Grid

Alt-S : Sweep the Circuit

Alt-T : Enter Tune Mode

Alt-V : Variable Component Edit

Alt-W : Edit Sweeps

Alt-X : Exit

Gain Plot : Solid Line is Gain, Dotted Line is Return Loss

Tune Mode : Tab Key to Advance to Next Variable Component

Esc Key to Leave Tune Mode

Press Any Key to Continue ...

The cursor frequency can be incremented or decrimented using

the "+" or "-" keys, respectively.  The  graphics cursor  will be

updated as  well as  the gain, phase, return loss and group delay

data at the bottom of the screen.

The Alt-A key is the active  sweep toggle.   If  the current

sweep is  #1, then  Alt-A will  change it to #2, clear the screen

and re-sweep the circuit using sweep #2.

The Alt-D and Alt-H  keys perform  opposite functions.   The

Alt-D key  doubles the  frequency step  size while  the Alt-H key

halves it.  QCkt doesn't respond to either  of these  keys in any

way - it just quietly updates the step size.  When the circuit is

re-swept (using the Alt-S  key, for  example), QCkt  will use the

new  step  size.    The  plot sweep time and resolution will vary

accordingly.

The Alt-F key allows the user  enter a  new cursor frequency

directly instead  of stepping  to it  with the  '+' or '-' keys.

When hit Alt-F, a window opens up and you're asked to enter a new

cursor frequency.  When you hit return, the window will close and

the screen will be updated to reflect the new cursor frequency.

The Alt-G key allows the user to edit the global parameters.

This allows  the user  to change component Q values, transmission

24

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

line parameters,  system  impedance,  etc.  quickly  and  see the

results almost immediately.

The Alt-N  key repaints  the screen  with the current cursor

frequency but does not re-sweep the circuit.  This is useful when

the screen  has gotten  messed up  or too cluttered with data and

markers.

The Alt-S key repaints the screen and re-sweeps the circuit.

Like Alt-N, it retains the current cursor frequency.

The  Alt-T  key  allows  the  user to enter Tune Mode.  This

causes several things to happen:  First, the  bottom line  of the

screen is  replaced by  a prompt  asking you to enter a new value

for one of the variable components.

At this point, you can enter a new  value for  the specified

component and hit return.  The circuit will then be re-swept with

the new component value.

If you hit the tab key  at  this  point,  the  next variable

component will be displayed and you can change the value of that.

If you hit the Esc key or Alt-X, you will exit tune mode.

The second  thing that  happens when  you enter tune mode is

the program changes from  the  normal  single-sweep  mode  to the

re-sweep  mode.    Normally,  QCkt  does not re-sweep the circuit

unless told to via the Alt-S  (for  Sweep)  key.    However, when

changing the  variable components,  it's convenient  to have QCkt

re-sweep the  circuit every  time you  change a  component.  When

this alternate  mode is  active, QCkt re-sweeps the circuit every

time one of the  variable components  is adjusted.   To  keep the

screen from  getting too  cluttered, QCkt will repaint the screen

after four sweeps. Also, if you're  plotting gain  and enter tune

mode, the  return loss is no longer plotted (this also helps keep

the screen clear of clutter).

The Alt-V key allows the user to view and edit  the variable

components.    See  chapter  7  on  variable  components for more

information.

The Alt-W key leads to the  edit sweep  menu.   The user can

then twiddle with the sweeps or change the active sweep.

The Alt-X  key is  the exit key.  Pressing this key will get

you back to the QCkt main menu.

25

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Chapter Seven - The Smith Chart

"Feel cheated ... "

Professor Milton Kult,  University  of Akron,

retired, commenting  on the  fact that he had

only 3  weeks in  which to  explain the Smith

Chart and it's uses.

You can get to the Smith Chart plotting routine via the QCkt

main menu option #6.  As with the X-Y plots, the program  asks if

you  want  to  sweep  the  current  circuit (which is the default

option), edit  the  variable  components  or  change  the current

frequency  sweeps.    Enter  the appropriate number or hit return

alone to accept the default.

QCkt then asks if you  want  to  keep  the  currently active

sweep.  Answer appropriately and hit return.  When you do, you'll

be at the Smith Chart plotting screen.

The Smith Chart takes up most  of the  screen.   If it isn't

round on  the particular  monitor you're  using, quit now and run

the Aspect.Com program provided  (See Appendix  E below  for more

information).

The Smith  Chart plotting routine behaves very much like the

X-Y  plotting  routines  described  in  Chapter  5.    As before,

information dealing with circuit behavior at the cursor frequency

is presented but, this time, it's on the left side  of the screen

instead of the bottom.

Starting  at  the  top  of  the  screen,  we have the cursor

frequency in MHz.  Below  that  is  the  complex  input impedance

given in  Ohms, followed by the VSWR and return loss.  Then comes

the reflection  coefficient  (labelled  Gamma),  the  Smith Chart

normalization impedance  (usually 50 or 75 Ohms) and the state of

the variable  components  at  the  present  time.    Finally, the

current default file is given at the bottom of the screen.

Also on the bottom of the screen is the label:

"R = 0.5, 1, 2"

This  is  a  reminder  that the three constant-resistance circles

26

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

displayed on the Smith chart are the R = 0.5, R = 1 and R = 2 Ohm

circles.

27

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

As in  the X-Y plots, you can get to a help screen by typing

Alt-I (for Information).  The help screen is reproduced here:

Smith Chart Help - QCkt (Version X.XX)

-----------------------------------------------------------------

+/-   : Increment/Decrement Freq

Alt-A : Active Sweep Toggle

Alt-C : Draw Constant VSWR, Constant Gamma or Matching Circle

Alt-D : Double Freq Step Size

Alt-F : Enter New Cursor Frequency

Alt-G : Edit Globals

Alt-H : Halve the Freq Step Size

Alt-I : This Help Screen

Alt-N : Draw New Smith Chart

Alt-S : Sweep the Circuit

Alt-T : Enter Tune Mode

Alt-V : Variable Component Edit

Alt-W : Edit Sweeps

Alt-X : Exit

Tune Mode : Tab Key to Advance to Next Variable Component

Esc Key to Leave Tune Mode

Press Any Key to Continue ...

These keys are exactly the same keys used by the X-Y plot to

get things  done.   I refer  you to  Chapter 5 for information on

these keys.  However, there is one key that has been added  - the

Alt-C key.

The  Alt-C  key  allows  you  to  draw either constant VSWR,

constant  Gamma  (reflection  coefficient)  or  matching circles.

Pressing the Alt-C key will produce the following menu:

Smith Chart Circles

-----------------------------------------------------------------

1.  Constant VSWR Circle

2.  Constant Gamma (Reflection Coefficient) Circle

3.  Matching Circle

99.  Return

-----------------------------------------------------------------

What'll It Be : (1) ...

The constant VSWR and constant Gamma circles are pretty much

28

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

the same thing, but the data is entered in different units.  QCkt

will  prompt  you  for  either  the  VSWR  or  Gamma value you're

interested in, then return you to the Smith Chart plotting screen

and draw  the specified  circle on  the chart.  In case you don't

know, the circle's center is at the center of the Smith Chart and

it's radius  is determined  by the  value you entered for VSWR or

Gamma.  Anyway, these  circles are  useful if  you're designing a

circuit which  must meet some VSWR specification.  (They are also

useful as a teaching aid.  See the examples in Chapter 8 for more

information).

The matching  circle is another useful tool.  I refer you to

the examples (ExLMat.Ckt and  ExLMat2.Ckt)  and  to  P.H. Smith's

book  listed  in  the  reference  section  (Appendix  C) for more

information.

29

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Chapter Eight - Some Examples

To help you understand how QCkt works and  the best  ways to

use the  program, I've  included several  example circuits on the

disk with the program.

The example circuit files I've  included  are  named  in the

form EX*.CKT  - that is, they all begin with the letters 'EX' and

have the '.CKT' extension.

The following four circuits  are  100-MHz,  5-pole, low-pass

filters.   The first is a Butterworth, the second is a Chebychev,

the third is an elliptic filter  and  the  fourth  is  a Guassian

filter.    These  are  included  here to allow you to compare and

contrast the responses of  these  very  different  filters.   The

primary sweep  goes from  0 to 300 MHz to give you an idea of the

filter's  ultimate  rejection,  stopband  behavior,  etc.     The

secondary  sweep  goes  from  0  to  100  MHz and shows the basic

passband characteristics of the filter.

For comparison's  sake, we  will examine  the attenuation of

these filters at 250 MHz.

ExBWLPF.Ckt - This filter was designed to have a 100 MHz, 3-

dB cutoff frequency.  In the gain plot  using sweep  #1, note the

smooth,  monotonic  response  and  how  the  return  loss  gently

increases.   These characteristics  are typical  of a Butterworth

filter.

Alt-F your  way to  250 MHz  and read  the attenuation.  You

should read about 38.3 dB of rejection.

Examine the close in response by  toggling to  sweep #2 with

the Alt-A command.  Note the smooth roll off.

The  phase  and  group  delay plots show relatively friendly

performance.  The peak group delay (using the Alt-F and +/- keys)

is 8.2  ns at  93 MHz.  The minimum group delay is found at 2 MHz

and is 5.1  ns.    This  makes  the  group  delay  ripple  3.1 ns

throughout the passband.

Try increasing the Q-factors of the inductors and capacitors

to  10000  and  examine  the  effect  this  has  on  the filter's

performance.

The  Smith  Chart  plot  shows  the  input  impedance of the

filter.  A Butterworth filter will smoothly rotate  away from the

center of the chart and approach it's final value smoothly.

This particular filter appears as a low impedance out-of-band due

to the shunt capacitors on the input.

30

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

ExCbLPF.Ckt - This is  a 1-dB  ripple Chebychev  filter.  It

was designed to have a 3-dB cutoff frequency of 100 MHz.

The most interesting thing about the gain plot is the return

loss.   Instead  of  smoothly  rolling  up  like  the Butterworth

filter, the  Chebychev shows ripple in the return loss as well as

in the passband magnitude performance.

Note the attenuation at 250 Mhz  is 58  dB which  is 19.7 dB

better than  the Butterworth  filter.  The price of the increased

out-of-band rejection is ripple in the passband.

Examining the close in response with  the Alt-A  command, we

see  that  our  filter  does  not  really  have  1-dB  ripple  as

advertised.  It droops as  it  approaches  the  cutoff frequency.

This is  due to  the finite  Q of  our components.   Use Alt-G to

change both the inductor and capacitor Q to 10000 and resweep the

circuit using Alt-S.  The filter now behaves as the textbooks say

it should.  Change the capacitor Q back to 1000  and the inductor

Q to 250 before continuing.

Examining the  group delay plot, we see the peak group delay

is 20.1 ns at 96 MHz.  The minimum delay is 6.1 ns and it appears

at 29  MHz.  This makes the in-band ripple 14 ns which 4 times as

worse as the Butterworth filter.

The Smith Chart is interesting.  The input  impedance sweeps

around  the  chart's  center  several times before heading toward

it's ultimate value.  In this  case, the  ultimate impedance will

be high due to the series inductor at the input of the filter.

ExElLPF.Ckt -  The magnitude, phase and group delay plots of

an elliptic filter are all pretty interesting to look at.   These

filters are  really a  mess as far as their group delay and phase

response are concerned.   However, they  do exhibit  an excellent

magnitude response.

The example  filter was  designed for  a shape factor of 2:1

with 40-dB minimum of  attenuation in  the stopband.   The worst-

case  passband  VSWR  was  around  1.04:1 with transmission nulls

placed at  325  MHz  and  209  MHz.    The  nulls  are  caused by

components 2  and 4 - both are parallel-LC circuits placed in the

series path.

Sweep #1 goes to 500 MHz and allows  you to  see both nulls.

Alt-F your  way to  the nulls  (you may  have to use the +/- keys

also) -- I measured them at 210 and 324 MHz.

Note that the stopband attenuation never  rises above  40 dB

which  was  one  of  the  design  goals.    At  our  250 MHz test

frequency, I measured -40.3 dB of attenuation, a phase reading of

175.9 degrees and a group delay of 1.1 ns.

Elliptic  filters  can  have  a steeper roll off than either

Chebychev or  Butterworth,  but  the  price  you  pay  is  in the

stopband performance.   While  the Butterworth and Chebychev roll

off monotonically (ie. they never re-enter),  the elliptic filter

31

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

will re-enter  but is  guaranteed by  design never to exceed some

minimum stopband attenuation value.

Another price you pay for the improved amplitude response of

the  elliptic  filter  is  in  phase and group delay performance.

Near the cutoff frequency, the phase response begins to show non-

linear  behavior.    In  the  stopband, the phase and group delay

responses  are  hideous;  exhibiting   huge  jumps   due  to  the

transmission zeroes.

The  Smith  chart  shows  the  filters input impedance moves

smoothly from the center  of  the  chart  to  it's  ultimate low-

impedance value.

ExGaLPF.Ckt -  The Gaussian  filter is  a very low-Q filter.

It has a poor magnitude  rolloff  characteristic  but  it  is the

least offensive as far as phase and group delay distortion go.

Load ExGaLPF  and plot the magnitude response out to 300 MHz

(sweep  #1).    You  can  see  the  rolloff  is  pretty  poor, as

advertised.    At  250  MHz,  the attenuation is only -21.4 dB of

attenuation compared with 38 dB for the Butterworth and 58 dB for

the Chebychev.   The  phase at this frequency is 59.8 degrees and

the group delay is 1.8 ns.

Plotting the phase response shows  the  real  beauty  of the

Gaussian filter.     The response is remarkably linear throughout

the passband and partially into the stopband (to around 150 MHz).

Plot the group delay.  You'll see that it's rock-solid  until 125

MHz or so, then it gently rises outside of the passband.

Because  they  are  so  gentle  on  the  phase  of a signal,

Gaussian  filters  are  used  in  places  where  phase   must  be

preserved,  such  as  television  video.   Due to their low, flat

group  delay,  these  filters  are  also  used  in  fast sweeping

receivers and spectrum analyzers.

ExBPBRF.Ckt - This is a bandpass/bandreject filter I used in

a transmitting/receiving  system.   The receiver  operated at 650

MHz while  the transmitter  used the same antenna at 750 MHz.  In

order to  protect  the  sensitive  receiving  circuitry  from the

transmitter, I  designed a  front-end filter to pass 650 MHz with

presenting a high impedance to 750 MHz.

Sweep #1 goes from  450 to  950 MHz  and shows  the passband

centered around  650 MHz  and the reject band centered around 750

MHz.  The passband response is Chebychev with  0.5 dB  of ripple.

The insertion  loss at 650 MHz is 1.54 dB and the filter provides

83 dB of rejection at 750 MHz.  In practice,  these numbers were-

3  dB  at  650  and  -70  dB at 750 due to component Q problems--

change the inductor Q to 120 and see for yourself.  Sweep #2 goes

from 600 - 700 MHz to show the passband response.

Take sweep #1 out to 2000 MHz using the Alt-W key.  Note the

re-entrance at 1420 MHz.  This is there because this  filter uses

32

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

a capacitor as a coupling element (ie. element #4).  If we change

this capacitor  to  an  inductor,  the  re-entrance  problem will

disappear.

Try it  -- change element #4 to a 70 nH inductor.  This will

cause the bandpass center frequency  to  shift  slightly  and the

hump at  1420 MHz  will disappear.  Tune component #2 slightly to

bring the filter back onto  frequency  (using  the  Alt-T  key to

enter tune mode).

The Smith  Chart shows the Chebychev response clearly.  Note

how the filter approaches a high impedance at  the 750  MHz notch

frequency.   With in  capacitor in place as the coupling element,

you can also see the re-entrance occur at 1420 MHz.

ExLMat.Ckt -  This is  an exercise  in interactive matching.

The circuit  is a simple L-matching network between two resistive

elements: R-Source = 50 Ohms and R-Load = 1000 Ohms.  We want the

match to be perfect at 50 MHz.

Load  ExLMat.Ckt  from  the  disk  and get a printout of the

circuit using the Shift-PrtSc key.  Draw a picture of the circuit

and  label  the  components.    The  circuit  consists of a shunt

capacitor across  the  1000-Ohm  resistor  followed  by  a series

inductor.   Next, look at the variable components menu to confirm

the shunt  capacitor  is  variable  component  A  and  the series

resistor is variable component B.  Write this on your diagram.

The purpose of this little exercise is to match the 1000 Ohm

resistor to 50 Ohms at 50  MHz.   We'll do  this using  the Smith

chart and variable components.

Enter  the  Smith  Chart  plotting  routines  and  sweep the

circuit.  Since the component values  are all  0, the  plot isn't

very interesting.   Set  the cursor frequency to 50 MHz using the

Alt-F key.

Now, enter the tune  mode  using  the  Alt-T  key  and start

adjusting component  A (the  capacitor in  parallel with the 1000

Ohm resistor).  Note  how  adding  capacitance  moves  the cursor

point in  a circle.   Adjust the capacitor until the cursor falls

onto the R=1 circle.    Now,  start  adjusting  component  B, the

series inductor,  until the  cursor falls  onto the center of the

Smith Chart.  If  you're  having  trouble,  I  made  the inductor

(component B) to be 0.68 uH and the capacitor (component A) to be

14 pF.

Alt-X your way out of the Smith Chart  plotting routines and

plot the  magnitude response  of the  matching network (main menu

option #3).   You'll  see that  the circuit  is matched perfectly

only at  50 MHz.   Using the Alt-F and +/- keys, we find the 3-dB

33

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

points of the match are at 37 and 61 MHz.  This is a bandwidth of

24 MHz or 48%.

ExLMat2.Ckt -  This is  another matching  example but with a

twist.  The bandwidth  of the  L-match circuit  of ExLMat.Ckt was

about 48%  but we'd like to try to do better (ie. make it wider).

This circuit consists of two  L-matching  networks  in  series to

produce a wider bandwidth.

As  before,  we're  trying  to  match a 50-Ohm R-Source to a

1000-Ohm R-Load but this time  we're  going  for  an intermediate

match  to  224  Ohms  (224  Ohms  is the geometric mean of 50 and

1000).

Load ExLMat2.Ckt and get  a  print  out  of  it  as  you did

before.   Then, go to the Smith Chart plotting routine.  Variable

component A is a capacitor in  shunt with  the 1000-Ohm resistor.

Component B is a series inductor.  Components C and D are a shunt

capacitor and  series inductor,  respectively.   Finally, we have

the 50-Ohm source resistor.

First, we  have to  find out  where 224 Ohms is on the Smith

Chart.  This is easy enough to do -- Use Alt-G to edit the global

variables and  change the  load resistance to 224 Ohms.  Then re-

sweep the plot using Alt-S.  Since all of our  components started

with zero  value, we  will see  a mark  on the  Smith Chart which

represents the load resistor (224 Ohms).  Make  a mental  note of

where 224  Ohms is on the Smith Chart.  Now, Alt-F your way to 50

MHz and Alt-T into  the tune  mode and  adjust components  A (the

capacitor in  shunt with  the 1000-Ohm  load resistor) and B (the

series inductor) until the impedance looking into the  circuit is

as  close  to  224  +  j0  Ohms as possible.  Then, start playing

around with components C and D to move the 224 Ohms to 50 Ohms.

The values I got for this circuit were 5.9 pF (component A),

1.33 uH  (component B),  26.6 pF  (component C) and 0.295 uH (for

component D).  Now, plot the  magnitude response  of the matching

circuit.   Note that the 3-dB bandwidth of the match goes from 31

to 74 MHz.  This is  43 MHz  or 86%.   The  bandwidth of  the two

element match of ExLMat.Ckt was 43%.

34

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Chapter Nine - How QCkt Works

I've included  this chapter  for the more curious of you out

there.  Several people have asked how QCKt does what it does.

QCkt  makes   use  of   a  technique   called  the  Modified

Tack-Hammer method,  fully described  in Heyward's excellent book

Introduction to Radio Frequency  Design.   This method  is fairly

crude,  but   it  isn't  very  complicated,  it's  intellectually

satisfying and it gets the job done.

QCkt starts with the load resistor  and assumes  that it has

one  volt  across  it.    The  program then travels up the ladder

toward the source  resistor,  evaluating  the  impedance  of each

element as  it goes.  QCkt calculates the impedance seen looking

toward the load at every step  and adjusts  the voltage according

to this  impedance.   The analysis  program stops when it gets to

the source resistor.  This process generates the  input impedance

and the  voltage present  at the  source.   QCkt then adjusts the

load/source  voltage  ratio  according  to  the  load  and source

resistors to get meaningful numbers to plot.  This entire process

is repeated for every evaluation frequency.

35

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Chapter Ten - QCkt Limitations

When writing a program like QCkt, there are  many trade offs

and considerations  to make.  I originally wrote this program for

my own, personal use and  I  feel  the  compromises  I  made were

reasonable.  They were made either because I don't like answering

the  same   question   over   and   over,   to   avoid  run-time,

divide-by-zero  errors,  for  the  sake  of  speed  or because of

personal taste.

In any case, if the program  starts behaving  funny or gives

you an  answer you  don't like,  consult this chapter first.  You

may have run into one of my compromises.

Frequency Sweep - Any time the program tries to evaluate the

circuit at 0 Hz, it will be evaluated at 1 Hz.

Step Size  - QCkt  takes the absolute value of any step size

that you enter.  If you enter a  0 for  the step  size, Qckt will

default to the value

Abs(Stop Freq - Start Freq)/100

which will give 101 points.

Component Values  - Generally,  you may  enter any component

value that you want  -  positive,  negative  or  zero.   Positive

values are obviously OK.  Mathematically, negative components are

no problem for the evaluation software but they may screw  up the

plotting routines.  They won't crash, but they may try to plot to

strange places on the screen.

Any component specified to be  zero  is  assumed  to  have a

value of 1E-12.

Other parameters  that are  not allowed  to be zero (besides

components) are: system  impedance  (Z0),  load  resistor, source

resistor,  inductor  Q,  capacitor  Q  and the frequency at which

electrical lengths are specified.  These also default to 1E-12 if

you enter a 0.

Number of  Components -  The maximum number of elements QCkt

36

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

allows is 98.  This is pretty much  a tradeoff  between amount of

memory used  and realizable  circuits.   It's fairly difficult to

design a passive  circuit  with  98  elements  that  will perform

properly.

Group  Delay  -  Group  delay  is  calculated  by taking the

derivative of  the  phase  plot  with  respect  to  frequency and

changing the  sign.   QCkt takes the derivative by evaluating the

circuit at the cursor frequency and at the cursor frequency minus

the  step  size.    It  then  subtracts one phase from the other,

adjusts the sign appropriately then divides  by the  step size to

obtain the derivative.

This means  that the  value of  the group  delay will change

slightly depending upon the step size.   For  more accurate group

delay data, decrease the frequency step size.

Hardcopies -  To make  hardcopies of  your circuit, list the

circuit and Shift-PrtSc the  data  over  to  your  printer.   The

circuit listing software prints out one screen at a time to allow

you to get the entire circuit.

You can also use the Control-PrtSc  option on  your computer

to send data from your screen to your printer.

To make  hardcopies of  the graphics screens, it's necessary

to have loaded the Graphics.Com program that came  with your DOS.

Simply  type  "Graphics"  from  the  DOS prompt prior to starting

QCkt.  You can then Shift-PrtSc your plots out to your printer.

If there is a lot of  interest in  QCkt, I  will upgrade the

printing utilities to something a little more sexy.

Variable  Components  -  If  a  component  is set up to be a

variable component (Component A, for example)  and it  is changed

from  the  Examine/Modify  main  menu  option,  QCkt  will  clear

Component A and declare it undefined.

Compatibility - Due to the  Turbo  Pascal  compiler  used to

generate   QCkt   and   QCkt-87,   their   file   structures  are

incompatible.  That is, QCkt cannot read or  write QCkt-87 files.

Likewise, QCkt-87 cannot read or write QCkt files.

37

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Appendix A - Technical Specifications

QCkt  was  written  using  the Borland Turbo-Pascal compiler

V3.0.  It is primarily Pascal  with a  small amount  of assembler

thrown in.

The QCkt  program is  actually made up of two modules:  QCkt

is the main module and handles user the input/output functions as

well as formatting data for the FarCode module.  FarCode contains

the circuit analysis and plotting routines.

QCkt.Com is the main module.   However, since  I needed more

than  the  64-k  of  code  that  Turbo  Pascal allows, I load the

FarCode.Com module on the heap of QCkt when QCkt is initializing.

38

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Appendix B - Acknowledgements

This program and manual would not have been possible without

the support, love and/or publications of the following people:

My wife and newborn son, Christopher, who allow me to fiddle

with my computer until all hours of the morning,

John Cooper, Terry Fry, Tom Vito, France Bolei  and the many

others I've  spent happy hours with discussing/arguing about some

silly aspect of circuit theory or computer programming,

All of the people on the  computer bulletin  boards who have

donated their precious time and software to the public domain.

39

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Appendix C

References - Credit Where It's Due

The  following  are  a  list  of  a  few  of  the books I've

consulted in the course of writing this program and manual:

Williams,  Arthur  B.,  Electronic  Filter  Design Handbook,

McGraw-Hill Book Company, New York, NY, 1981, ISBN 0-07-070430-9.

Hayward,  W.H.,  Introduction  to  Radio  Frequency  Design,

Prentice-Hall,   Inc.,   Englewood   Cliffs,   NJ,   1982,   ISBN

0-13-490021-0.

Jordan, Edward C., Ed., Reference Data for Engineers: Radio,

Electronics,  Computers,  and  Communications,  Seventh  Edition,

Howard  W.  Sams  &  Co., Inc., Indianapolis, Indiana, 1985, ISBN

0-672-21563-2.

Smith, Phillip  H.,  Electronic  Applications  of  the Smith

Chart,    McGraw-Hill Book Co., New York, New York, 1969, Library

of Congress Catalog Card Number 69-12411.

40

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Appendix D - Component Summary

This appendix  is meant  as a  quick reference  guide to the

components.    It  is  essentially  a repeat of the data found in

Chapter 3 on Entering and Saving a Circuit.

1. Capacitors - The value  of  all  capacitors  in this

program  are   currently  given  in  picofarads.    All

capacitors have the same global Q  value which  is held

constant  with   frequency.    The  default  value  for

capacitor Q is 1000.

2. Inductors -  The  value  of  all  inductors  in this

programs  are  currently  given  in microhenries.  Like

capacitors, all inductors have the same  global Q value

which can be (and usually is) different from the global

capacitor Q.  The default value for inductor Q is 250.

3. Resistors - The values of all resistors  are entered

in Ohms.

4.  Parallel  LC  -  An  inductor  and capacitor hooked

together in  parallel.   As before,  the capacitance is

given  in   picofarads  while   the  inductance  is  in

microhenries.  Global capacitor  and inductor  Q losses

are applied to this combination.

5. Series  LC -  As in  #4, but  the two components are

connected in series.

6. Parallel RC - A resistor and capacitor  connected in

parallel.

7. Series  RC -  A resistor  and capacitor connected in

series.

8. Parallel RL - A resistor  and inductor  connected in

parallel

9. Series RL - A resistor and inductor in series.

10. T-Line - This is a lossless transmission line.  The

user enters a unique characteristic impedance (in Ohms)

41

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

and electrical  length (in degrees).  If the line is in

shunt with the circuit, the user  is asked  if the line

is open-  or short-circuited.  The electrical length is

the number of degrees the line appears to be at a given

frequency.    The  frequency  is  specified as a global

value and it is the same for all of the T-Lines entered

in this program.  The default is 100 MHz.

11. Parallel  Resonator -  This is simply a parallel LC

circuit (see  item #4  above) specified  in a different

way.   Instead of  asking for  a value  of L and C, the

program asks for a value of  L (in  microhenries) and a

center frequency  (in MHz).   The parallel LC will then

be made to resonate at  the  given  frequency.   Global

capacitor  and   inductor  Q's   are  applied  to  this

component.

12. Series Resonator -  The same  component as  in item

#11,  except   that  the  inductor  and  capacitor  are

connected in series.

13. Defined Impedance Plane - This is an operation more

that it  is a component.  If one of these is present in

the circuit, the Smith Chart and return loss plots will

reflect the  impedance of  the circuit looking from the

impedance plane toward the load.   The Smith  Chart and

return loss  plots will not reflect the input impedance

as the source sees the circuit.

14.  Duplicate  an  Existing  Part  -  This  is  a very

convenient   operation   which  allows  any  previously

defined part (ie.  a  part  with  lower  number)  to be

duplicated.   This is very convenient for some types of

filters and  matching  circuits  which  exhibit  a high

degree of symmetry.

42

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Appendix E - Running Aspect.Com

One of  the problems  I had  when writing  QCkt was that the

Smith Chart  would  come  out  elliptical  on  some  monitors and

circular on  others.   For example,  the Smith Chart on my Zenith

monochrome monitor  at home  is fine,  but it  turns out  to be a

narrow ellipse on the IBM monitor at work.

Usually  you  can  solve  this  problem by fiddling with the

controls on your monitor.  Sometimes  you can't.   If  you can't,

I've included a program called Aspect.Com on the disk.

Aspect.Com draws  a horizontal  line on  the screen and asks

you to get out a ruler, measure the  line's length  and enter the

number.      Aspect  then  draws  a vertical line and repeats the

request.

The unit you use to measure the line is immaterial, but it's

easiest to  use a  centimeter scale.  That way, you don't have to

convert fractional inches to  decimal before  entering the number

into the computer.

Since  Aspect.Com  knows  how  many  pixels it lit up on the

screen when it drew the two lines and you've told it how long the

lines were  physically, it  can figure out a number of pixels per

unit length for both the vertical and horizontal  directions.  It

then divides  the horizontal number by the vertical number to get

a ratio.  This ratio is  stored in  a file  on the  default drive

under the name Aspect.Dta.

When  QCkt  first  comes  up,  it  loads Aspect.Dta from the

default drive  if it's  present (if  it isn't  present, QCkt just

uses an internal default value for the aspect ratio).  All of the

data heading for the Smith Chart screen is then massaged  by this

aspect value before it hits the screen.

So, the  circles are  drawn so  they look round and the data

plotted by the Smith Chart routines is plotted accurately.

43

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Appendix F

-----------------------------------------------------------------

QCkt - Circuit Analysis Program - Order Form

-----------------------------------------------------------------

RadioWare

P.O.Box 2016

Columbia, Md   21045

PRICE      PRICE

DESCRIPTION                       QUANTITY     EACH     EXTENDED

-----------------------------------------------------------------

QCkt program disk - Try Before     ______     $10.00    $_______

Buying - Latest version,

complete program with manual

and examples on disk, NO

technical support.

QCkt - Registered Owner -          ______     $25.00    $_______

Includes a free update to the

NEXT version - manual and

examples on disk - All the

technical support you need.

QCkt - Upgrade to Registered       ______     $15.00    $_______

Owner - If you all ready sent

in $10.00.

QCkt - Upgrade to Newest           ______     $10.00    $_______

Version - Registered Owners

ONLY.

Subtotal    $_______

5.0% Maryland State Sales Tax        $_______

(Maryland Residents Only)

Total       $_______

Please make checks (U.S. funds only) payable to: RadioWare

Name_______________________________________ Date ________________

Address__________________________________________________________

_________________________________________________________________

City________________________ State__________ Zip Code____________

Description of Computer System___________________________________

44

QCkt - The Quick Circuit Analyzer - (c) 1988 RadioWare, Inc.

Commercial users of QCkt MUST  register  their  copies  at a

price of  $25.00 each  for the  first 20 copies, $20.00 for every

copy  thereafter.    In  addition,  a   site  license  permitting

unlimited copying  within the  licensed institution is available.

Please write for details.

45