TN210-2.DOC

THENET PLUS VERSIONS

************************

Version  2.00  was  the prototype test node.  The maximum number  of  calls

listed in the Heard list was 10 over a 10 minute period.

Version  2.01  was the first  "general release" of TheNet Plus.  The  Heard

list  was  changed to a maximum of 20  calls listed over a 15 minute period.  A

parameter  was added which  allowed the NodeOp to set the maximum number in the

Heard list to a value less than 20, if desired.

Version 2.02 was a non-released experimental node.

Version  2.03 was identical to version 2.01 with the exception of a bug fix

associated  with  Parameter  24.  In version 2.01, if  Parameter  24  (callsign

verification)  was turned off, it also disabled the N * function.  Version 2.03

corrects this situation.

Version  2.04  corrected  a problem caused by the fix in version  2.03  and

changed  Parameters  28, 29, and 30 to agree with the  SETPLUS utility and  the

documentation.

Version  2.05  added several new NodeOp convenience features.  One  was  to

have  the  STA LED light when someone connected to the node.  It was felt  this

would  assist the NodeOp in servicing his equipment while at the site.  Another

new feature  was to add three SYSOP KEY commands, MARK, SPACE and DIDDLE  which

keys  the transmitter and turns on appropriate alignment tones.  Also added was

an ON  - OFF remote control capability.  One of the node command responses  was

changed  to "Host busy", instead of "Host table full" when a user attempted  to

connect  to the Host (a disallowed function when the Host port is active).  The

"Not  Found"  response  to  an unknown node now indicates  "Not  Found:   <node

alias>" which purpose is to provide the user an "assurance prompt".

Version  2.06 corrected a long-standing routing display anomaly.

Version 2.07 added more network control functions to the node.  The STA LED

function  was  modified  from  indicating  node activity to now lighting when a

level 4 circuit (at the node) was either attempted or established.   The number

of SYSOP   adjustable parameters   were  reduced from 31 to 13.  However all 31

parameters were still settable with the SETPLUS utility.

Version 2.08 provided  enhanced network  control functions and improved the

password routine.  The number of SYSOP settable  parameters were increased from

13 to 16.   A  version 2.08B was  provided, the only  difference between it and

2.08 were that aliases AND callsigns appeared in the routes list.

Version 2.09  was a developmental model.  Several new features were tested.

One variation had the user table expanded from a total of 25 to 50 up and down-

links.  This version was released for DXPacketCluster beta testing.

With  version 2.10, three sub-versions are introduced.    These are 2.10st,

2.10dx, and 2.10tm.  2.10st is the normal, or standard version intended for LAN

and  backbone  trunk use.    All variations have an auto-reconnect feature that

allows users  who are "byed" or  retryed  out of a connect to  be automatically

returned to the next version 2.10 node along the circuit.   The  procedure  for

changing parameters is simplified.   HDLC code changes causes transitions to be

sent, instead of flags, during the TXD period.  This allows the target receiver

to react faster to incoming frames.

ALL NODE DIGIPEAT FUNCTIONS ARE REMOVED.   With few exceptions, digipeating

through a node adversely impacts normal network flow.  Sacrificing the digipeat

function  to  gain  programming  space  for the new  features was the resultant

trade-off.    For those requiring a digi function, version 2.08 should be used.

Parameters  for HEARD list length and DIGIPEAT are  eliminated.   Transport

parameters are SYSOP settable and the string total is 23, up from 16 in version

2.08.     Source and destination thrulinks are identified in the USERS display.

STA  LED  now  lights  on either of  thrulink (L3)  or  circuit (L4)  connects.

ROUTES display shows both alias and callsigns of neighbors.  2.10dx is the same

as 2.10st  with exception of user table size being expanded from 25 to 50.  The

dx version is intended to serve a DXPacketCluster.  2.10tm is designed for site

telemetry applications (requires daughterboard construction).

NOTE:  While 2.10tm is available, the development is on-going.  Rather than

delay distribution of TN2.10 until a low cost  "representative"  site telemetry

package is put together and debugged,  2.10tm is not included with this distro.

Approximate parts cost for the daughterboard is  $70.00.   Those  interested in

this  application   will  receive  the latest  tm ROM image  and schematics  in

exchange for a SASE 360K disk and mailer.  See file TN210_21.DOC for additional

details.