Problem description:

Temperature  field   in  the  stator  tooth  zone  of  power

synchronous electric  machine. Domain is a segment of stator

transverse section.  Two armature  bars laying  in the  slot

release ohmic loss. Cooling is provided by convection to the

axial cooling duct and both surfaces of the core.

Geometry is as follows:

Slot height = 0.074m;

Slot width = 0.02m;

Bar height = 0.029m;

Bar width = 0.015m;

Wedge height = 0.005m;

Stator inner diameter = 0.9m;

Stator outer diameter = 1.112m;

Cooling duct diameter = 0.015m.

Parameters:

Specific copper loss = 3.6E+05 W/m**3;

Heat conductivity of steel = 25 J/(Km);

Heat conductivity of insulation = 0.15 J/(Km);

Inner stator surface:

heat transfer coefficient = 250 W/(mK);

temperature of contacting air = 20C.

Problem is plane-parallel.