Problem description:

Magnetic  system   of  a   loudspeaker  including  cylindric

permanent magnet. Magnetic circuit is made of steel.

Model geometry:

System height = 0.05m;

Permanent magnet height = 0.03m;

Outer radius = 0.05m;

Permanent magnet radius = 0.025m;

Air gap = 0.006m.

Parameters of material:

Magnetic reluctivity of air = Mu0;

Magnetic reluctivity of steel = 1000 Mu0;

Magnetic reluctivity of magnet = 625 Mu0;

Coercive force of magnet = 500000 A/m;

Problem is  axysimmetric.  Problem  domain  is   surrounded

by the sphere possessing zero Dirichlet boundary condition.