{ Qinit.doc - documentation for QWIK Qinit                  ver 4.1, 05-01-88 }

{                                                                             }

{ Qinit initializes all the variables needed for the Q- procedures.  And      }

{ specifically checks for ALL IBM video equipment including dual monitors.    }

This file will help you figure out how the new IBM PS/2 systems are config-

ured as well as original equipment.  First, it is helpful to have a list of

all the possible Alphanumeric (A/N or text) modes.  QWIK was not designed

for the All Points Addressable (APA or graphics) modes.  Qinit detects dual

monitors and saves both the active and alternate display device codes.

TABLE 1:  Hardware Specific Video Mode Characteristics

Mode Format Segment Display Box  MDA CGA EGA MCGA VGA PCjr PCC 3270 HGC MaxPage

---- ------ ------- ------- ---- --- --- --- ---- --- ---- --- ---- --- -------

0,1  40x25  B800:0  320x200 8x8       x   x   x    x   x    x              7

320x350 8x14          x        x                       7

320x400 8x16              x                            7

360x400 9x16                   x                       7

2,3  80x25  B800:0  640x200 8x8       x                x    x              3

640x200 8x8           x        x                       7 *

640x350 8x14          x        x                       7 *

640x400 8x16              x                            7

720x350 9x14                                x          0+

720x400 9x16                   x                       7

7    80x25  B000:0  720x350 9x14  x                         x   x    x     0

720x350 9x14          x        x                       7 *

720x400 9x16                   x                       7

640x200 8x8                             x              3

Legend:

Format  - Characters per row by the number of rows in the data area.

Segment - Address of the first character on page 0 of the display buffer.

Display - The pixel resolution for the data area excluding the border,

horizontal by vertical.

Box     - The pixel resolution for each character, horizontal by vertical.

MDA     - Monochrome Display and Printer Adapter

CGA     - Color Graphics Adapter

EGA     - Enhanced Graphics Adapter

PGC     - Professional Graphics Controller

MCGA    - Multi-Color Graphics Array

VGA     - Video Graphics Array

PCjr    - PC Junior

PCC     - PC Convertible

HGC     - Hercules Graphics Cards - HGC, HGC Plus, and InColor Card

3270    - All IBM 3270 PC adapters

MaxPage - 0-based highest page number; e.g. 7 means there are 8 pages.

MD      - 5151 Monochrome Display

CD      - 5153 Color Display

ECD     - 5154 Enhanced Color Display

Notes:

1. The 0 and 2 modes suppress color burst only on composite displays (not

RGB) only for CGA and EGA.

2. The PS/2 model 25 and 30 have an integrated MCGA.  The model 50 and

above have an integrated VGA.

3. The 8514 High Content Color Display along with the 8514/A adapter

produces a superset of the VGA for APA, but there is no differences in

the A/N modes to the VGA when the adapter is in "VGA" mode.  See IBM

documentation for Advanced Function Mode.

4. Reduced MaxPage to 3 if EGA only has 64K graphics memory installed for

modes marked "*".  MaxPage of 7 is for 128K or more.

5. The PCC can have either an alternate MDA or CGA.  The LCD (model 5140)

can emulate either the MDA or CGA modes, but the MDA mode is 640x200.

6. I currently have no information on the PGC.

7. To relieve cursor routine programming due to the multiple character box

sizes, the VGA has a cursor emulation mode.  Qinit turns on the mode

if PS/2 equipment is detected.  See CURSOR.DOC for emulation notes.

8. On the 3270 PC, dual monitors are not possible as they use the same

physical buffer space.  Qinit will determine if more than just page 0

is available for color displays.  Unless there is a special adapter,

there is only page 0.

Display Combination Code (DCC) - The PS/2 video BIOS has a new function

that simplifies equipment detection called the Read/Write Display

Combination Code.  Using interrupt $10 with AH = $1A00, the call will

return the Active Display Device in BL and the Alternate Display Device in

BH.  If the function is supported, it also returns $1A to AL.  The possible

Display Device codes (as assigned by IBM) are:

TABLE 2:  Display Combination Code

CODE  DESCRIPTION                       QWIK41 Constant

----  --------------------------------  ---------------

$00   No display                        NoDisplay

$01   MDA with 5151 monochrome display  MdaMono

$02   CGA with 5153/4 color display     CgaColor

$03   reserved                          DCC3

$04   EGA with 5153/4 color             EgaColor

$05   EGA with 5151 monochrome          EgaMono

$06   PGC with 5175 color               PgcColor

$07   VGA with analog monochrome        VgaMono

$08   VGA with analog color             VgaColor

$09 to $0A - reserved                   DCC9,DCC10

$0B   MCGA with analog monochrome       McgaMono

$0C   MCGA with analog color            McgaColor

$0D to $FE - reserved                   n/a

$FF   unknown                           Unknown

Conforming to DCC - No results are obtained for the DCC on anything other

than PS/2 equipment.  However, to be consistent, Qinit was reprogrammed

to conform to the DCC for ALL equipment.  To see if a CGA is in use,

simply check to see if ActiveDispDev=CgaColor.  Qinit only sets the

parameters for the active display.  Although the alternate display device

code is saved, it is only for testing purposes.  If you change text modes

in a running program, Qinit should be executed again.

HavePS2 - Qinit sets HavePS2 to true if the DCC is supported.  This means

| that the program has detected a PS/2 video card whether it is a standard

Model 30, a PS/2 Display Adapter on an IBM XT, or the like.  It also

means that either MCGA or VGA is present, but not necessarily active.  To

know which, just check the DCC.

Have3270 - If Qinit detects 3270 PC equipment/software, this variable is

set to true.  In addition, the ActiveDispDev is either MdaMono or

CgaColor.  Note: There may or may not be graphics capability in either

case; Qinit is not meant to detect graphics.  ActiveDispDev3270 may be

one of the following:

TABLE 3:  3270 PC Display Combination Code

CODE  MONITOR        ADAPTER                    QWIK41 Constant

----  -------------- -------------------------  ---------------

$00   5151 or 5272   for 5151 or 5272

$01   3295           for 3295

$02   5151 or 5272   for 5151 or 5272 plus

XGA graphics

$03   5279           3270 PC G

$04   5379 C01       3270 PC GX

$05   5379 M01       3270 PC GX

$06   Reserved

$07   Reserved

$FF   Unknown, not a 3270 PC                    Unknown

EGA Switches - By checking the value of this byte, you can determine the

monitor connected to the EGA, the alternate video system, and the start

up default.  The byte is a copy of how the dip switches are set on the

card where on=0 and off=1.  The primary is the default.  When the ECD is

| set for 640x200, it is emulating the CD including the cursor mode.  Qinit

| now directly tests for the alternate device rather than assuming it.

TABLE 4: EGA dip switch configuration

SWITCH  VALUE  PRIMARY               SECONDARY

------  -----  --------------------  -------------------

0000    0      MDA + MD              EGA + CD - 40x25

0001    1      MDA + MD              EGA + CD - 80x25

0010    2      MDA + MD              EGA + ECD (640x200)

0011    3      MDA + MD              EGA + ECD (640x350)

0100    4      CGA + CD - 40x25      EGA + MD

0101    5      CGA + CD - 80x25      EGA + MD

0110    6      EGA + CD - 40x25      MDA + MD

0111    7      EGA + CD - 80x25      MDA + MD

1000    8      EGA + ECD (640x200)   MDA + MD

1001    9      EGA + ECD (640x350)   MDA + MD

1010   10      EGA + MD              CGA + CD - 40x25

1011   11      EGA + MD              CGA + CD - 80x25

EGA Information - The byte located at $0040:$0087 has the following

information when the EGA (or VGA) is present:

TABLE 5: EGA information

BITS  DESCRIPTION

----  -------------------------------------------------

7     (0) Clear, (1) Keep video data after mode set

6,5   (00) 64k, (01) 128k, (10) 192k, (11) 256k, memory

4     Reserved

3     (0) EGA active, (1) EGA not active

2     (0) Disable,    (1) Enable wait-for-display

1     (0) CD/ECD,     (1) MD attached

0     (0) Active,     (1) Inactive CD emulation

PCC Exceptions - Since the PCC also does not support the DCC, a separate

code is used.  If the 5140 LCD is used in mode 7, the Active Display

Device is set to MdaMono which is close enough.  This set up can be

verified by testing if MaxPage=3.  The alternate display is found by using

interrupt $10 with AH=$15.  The result is saved in AltDispDevPCC.  Of

course the variable is undefined if a PCC is not used.  The possible types

are:

TABLE 6: PC Convertible Alternate Display Adapters

VARIABLE        MODEL  DESCRIPTION

-------------   -----  --------------------

AltDispDevPCC = $   0   No alternate adapter

= $5140   LCD

= $5153   CGA type

= $5151   Monochrome type

Dual Monitors - Qinit makes an attempt to detect for a second monitor by

| several means.  If no alternate cards like EGA or VGA are found, an

| alternate 6845 video chip (CGA, MDA, or Hercules) is checked.  If the

| chip is found, then further tests are made to find which card it could

| be.  The chip existence test is highly reliable.  If no alternate display

| device is found, then AltDispDev=NoDisplay.

| Hercules - Hercules cards are also detected.  If either the active or

| alternate DCC is MdaMono, which is found by verifying a responsive 6845

| video chip at the mono register port, then an attempt is made to find if

| any Hercules card is attached.  If no Hercules card is found, then

| HercModel=NoHerc; it is then assumed that just an MDA card is attached.

| This test is only run once, even if Qinit is executed again, because the

| test can take up to 1/3 of a second on slower computers.  The tests for

the Hercules cards are the ones recommended by Hercules Computer

Technology, which also admits that sometimes the tests will fail during

multi-tasking activity.  Hercules clones may not be detected by these

test, but the DCC will be correct.

TABLE 7:  HercModel - Hercules Model Code

CODE  DESCRIPTION                       QWIK41 Constant

----  --------------------------------  ---------------

0    No Hercules card                  NoHerc

1    HGC                               HgcMono

2    HGC Plus                          HgcPlus

3    Hercules InColor Card             HercInColor

System Identification - To get both the model and submodel, you must use

| interrupt $15 with AH=$C0 which only works on some computers.  The code

| was modified to properly exit on AT&T, Leading Edge, and other clones

| which have BIOS inconsistencies.  If the carry flag is set on exit, then

that interrupt is not supported, because there's no submodel number.  The

alternative is to get the basic computer model by directly accessing the

byte in memory at $F000:$FFFE.  The possible values are:

TABLE 8:  System Identification Codes

SysID Sub-Model  SYSTEM

----- ---------  ---------------------------

$FF    n/a      IBM PC

$FE    n/a      IBM PC XT

$FD    n/a      IBM PCjr

$FC    n/a      IBM PC AT (6 MHz, 01/10/84)

$00      IBM PC AT (6 MHz, 06/10/85)

$01      IBM PC AT (8 MHz, 11/15/85)

$02      IBM PC XT (286)

$04      IBM PS/2 model 50

$05      IBM PS/2 model 60

$FB    $00      IBM PC XT (256/640)

$FA    $00      IBM PS/2 model 30

$01      IBM PS/2 model 25

$F9    $00      IBM PC Convertible

$F8    $00      IBM PS/2 model 80 (16 MHz)

$01      IBM PS/2 model 80 (20 MHz)

| CPU Identification - A CPU detection routine has been included for Intel

| processors.  This routine is independent of QWIK and is useful for clones

| that do not recognize IBM's system ID scheme.  The idea came from Juan

| Jimenez as it appeared in Jan/Feb '88 Turbo Technix magazine.  The

| routine has been simplified for reduced code (only 42 bytes) and allows

| use of simple constant identifiers.  Therefore, this routine is optional

| to  QWIK and may be deleted from the initialization in QWIKxx.PAS if

| desired.

|    TABLE 9:  Intel CPU Identification

|    INTEL CPU MODEL NUMBERS  QWIK41 Constant

|    -----------------------  ---------------

|    Intel 8086/8088          Cpu8086

|    Intel 80186/80188        Cpu80186

|    Intel 80286              Cpu80286

|    Intel 80386              Cpu80386

The essential variables set by Qinit are:

VARIABLE      DESCRIPTION

------------  ---------------------------------------------------------

Page0seg      Segment for the base page, page 0.  This value may be

changed for multi-tasking environments.

Qseg          Segment for Q writing which is altered when changing

pages.

Qsnow         Wait-for-retrace (snow) while Q writing.

MaxPage       Maximum possible 0-based page number.

The other variables set or located by Qinit are for your reference to

help make decisions in your programs:

VARIABLE      DESCRIPTION

------------  ---------------------------------------------------------

VideoMode     Directly accesses the video mode code.

VideoPage     Directly accesses the video page number.

EgaRows       0-based number of rows on the screen for EGA or VGA.

EgaFontSize   1-based number of horizontal scan lines per character for

EGA or VGA.

EgaInfo       (Described above)

EgaSwithces   (Described above)

CRTcolumns    1-based number of CRT columns for any card or video mode.

CRTcols       Global variable equal to CRTcolumns.

CRTrows       1-based number of CRT rows for any card or video mode.

|   QnextOfs      0-based offset where next Q*More procedure will write.

HavePS2       Detects the presence of IBM PS/2 video equipment.

Have3270      Detects the presence IBM 3270 PC equipment/software.

CardSeg       Video buffer address the video "card".

CardSnow      Requirement for wait-for-retrace for the video "card".

SystemID      Code for the IBM computer in use.

SubModelID    Code for the sub-model of the IBM computer in use.

|   CpuID         Intel CPU identification code.

ActiveDispDev Code for the Active Display Device.

AltDispDev    Code for the Alternate Display Device.

AltDispDevPCC Code for the Alternate Display adapter on the PC

Convertible.

ActiveDispDev3270 Code for the Active Display Device on the 3270 PC.

CGA Snow - A recent discovery has shown that CGA cards do not need wait-

for-retrace in 40 column modes 0 and 1.  Qinit was programmed to

accommodate this.  For other hardware, you can change Qsnow to suit your

needs, but CardSnow should be left unchanged to save what Qinit detected.

References - For more information on the new IBM PS/2 system, you can get

the "IBM BIOS Interface Technical Reference Manual".  Other references

include:

IBM Personal System/2 Seminar Proceedings:

Volume 5, Number 2, Displays and Adapters, publication # G360-2678.

Volume 5, Number 4, Models 50, 60, 80, VGA, BIOS and Programming

Considerations, publication # G360-2747.

For more information on the IBM 3270 PC, you can get the the following

publications:

"3270 PC Application Development Considerations"

"IBM 3270 Personal Computer Programming Guide", Pub # SA23-0221

"IBM 3270 Personal Computer Control Program Reference", Pub # GA23-0232

As always, the above information is subject to change without notice per

IBM.  IBM is the trademark for International Business Machines Corp. (but

you already know that!)