Conventions

The  following  conventions are used throughout  this

document.

'Enter'     means the Enter key  or  the Return

key.

Type 'key'  means press the specified

key without 'Enter'.

Press  F?   means press  the  specified

function key without 'Enter'.

Enter       means type in the  specified

text and followed by 'Enter' key.

Select/Use an option:

use   cursor  keys  to  move   the

highlighted  option and  press  'Enter',  or

type the upper-cased character on the left.

GAF is not case sensitive, use either upper  or

lower cased key.

Using The Menu

The  GAF  system menu bar, at the top of the  screen,

is  the  primary user interface to the  system.   The

system menu bar contains following menu items:  File,

Run,  Display,  Option, Adapt, Method,  Segment,  and

Help.   Use the first character of each menu item  or

use  cursor  keys (right/left arrows) to  select  the

menu  item.  When a menu item is selected a pull down

menu  appears  to  provide  function  selection.   To

select a function item in a pull down menu either  by

the  character on the left of that item or  move  the

cursor  with  up/down arrow keys  and  then  "Enter".

When  a  function item is selected, further secondary

menu  selection or data entry dialog may be  provided

by   that  function.   Except  rule  selection,   all

secondary  menus  and  data entry  dialog  boxes  are

displayed  immediately beneath the  pull  down  menu.

GAF  uses rule text window for rule selection.  Press

ESC  in  pull down menu or secondary menu will return

to  system  menu  bar.  Press ESC under  system  menu

will  bring  up the Run pull down menu,  except  when

the  system menu was ESCaped from any of the  running

mode  (i.e.  from  adapting or  simulation).   Please

refer  GAF.DOC for details.  The following steps  are

typical  for  function selection under simulation  or

adapting.

press "ESC" to escape back to system menu

select desired function menu (see above)

select desired function (see above)

press "ESC" again under system menu to go back

Introduction

This  simple example demonstrates the application  of

the  fuzzy  logic  to control a motor  based  on  its

angular position feedback.  Lets assume that this  is

a  robot arm, and its arm extension limit (in angular

position)  is from 0 degree to 70 degree.   The  demo

contains following segments:

Control    robotarm.gaf

Feedbackrobotarm.dat

Evaluation robotarm.evl

Reference  robotarm.ref

The  control  segment uses a set of  fuzzy  rules  to

control  the  motor.  The inputs to the control  loop

are  angular position of the arm, position error, and

angular  speed  of  the arm.   The  output  from  the

control segment is the current to the motor.

The  feedback segment uses a set of data to  emulaste

the  reaction  of the robot arm (the  motor  and  its

sensors).   Outputs  of the feedback  loop  are:  the

angular  position and the angur speed  of  the  robot

arm.   Note  that  there  is a second  example  using

fuzzy  rules  to  emulate the  feedback  loop  please

refer to the file robotarm.fbk for details.

The   reference   segment  calculates   the   angular

position error and generates a flag POS_REG  for  the

above two segments.  This flag is used to enable  the

segment  or part of its logic.  The reference segment

can be scheduled in a lower rate if so desired.

The  evaluation  segment evaluates the  control  loop

for  the  application.   Based  on  fuzzy  rules   it

calculates  the EVAL_RESULT output variable  for  the

automatic adaptation.

Simulation

Run Simulation

After  start  the  demo, GAF displays  the  system

menu  at  the top of the screen.  Select run  menu

by  typing 'R', then select "simulation  mode"  by

either  type  'S' or move high lighted  cursor  to

"simulation   mode"  and  press  'Enter'.    After

simulation  mode  is selected,  GAF  displays  the

control  segment on the screen and waits for  user

input.   Please  refer to GAF.DOC for  details  of

the  display  windows.   Type  'F'  to  start  the

simulation.  You  should see the  trend  plots  of

inputs and outputs on the right and the rules  and

firing status on the left of the screen.

Initialize the system

Press  function key F7 to initialize  the  system.

The  system reset is defined by the INITIALIZATION

declaration  in all segments of the system.   Type

'F' again to run the simulation.

View the feedback loop

Now  switch  to the feedback loop by pressing  F3,

the  screen displays the feedback segment.   Press

F7  to  initialize the system, and then type  'F'.

Now  watch the plot of the inputs and the  outputs

for the feedback loop.

Stepping through

Let's  switch back to the control loop by pressing

F3  three  times, and initialize it  by  F7.   Now

press  'Enter'  to see the demo program  goes  one

step.   The program allows you to do one  step  by

'Enter' or multiple steps by typing 2 to 9, or  10

steps  by  typing  '0'.  Type '0'  and  watch  the

program  goes through 10 steps and note  different

rules  are fired.  At any time type 'F' will bring

you to the end of the plot frame.

Continuous mode

Press  'C'  to  run  the simulation  continuously.

You  can  stop  continuous mode  by  pressing  'C'

again  or ESCape (press ESC) back to system  menu.

While  in  continuous mode press F7 to  initialize

the system.

Change variable value

Stop  continuous mode by pressing 'C'.  Now  let's

change  the  simulation from raising  the  arm  to

lowering  the  arm.   Press F7  and  then  ESC  to

escape  back to system menu.  Press 'S' to  select

segment  menu.   The segment menu  allows  you  to

switch  segment and change characteristics of  the

segment.   Switch to feedback segment by selecting

"segment number" under segment menu, or use F3  to

switch.   When  using Segment  menu  to  select  a

segment,  GAF  displays all  segment  names  in  a

secondary  menu and prompt for selection.   Select

segment  menu again, and select "set input value".

GAF  pops all inputs in a menu beneath the segment

menu,  select "REFERENCE" input and enter its  new

value to 0.  Select segment menu one more time  to

change the input variable "POSITION" value  to  70

(from  "set  input value").  Now get back  to  the

simulation  by pressing ESC, press  F3  to  switch

back  to control segment, and start simulation  by

stepping  through  it.   Note that  you  can  also

change  the  "POSITION"  value  from  the  control

segment (segment 0).

View the plot

From  system menu type 'D' to select display menu.

The  display  menu  allows you  to  rearrange  the

display  windows.  Let's select "select plot  only

mode"  by  typing  '5', and then  escape  back  to

simulation.   GAF  should redraw the  screen  with

trend   plot  takes  most  of  the  screen.    Run

simulation by hiting F7 and then 'F'.

View the combined result

From  Display  menu select "select  whole  display

mode".   The  right portion of the  screen  splits

into  two  parts, the upper half shows  the  plot,

and  the  lower half displays the combined  result

for  the  output.  The combined result  shows  the

trapezoid   result  of  all  fired  rules   (rules

generate  non-zero output).  Rerun the  simulation

by  pressing F7, and then 'F'.  Now, the  combined

result  tells you about the firing results of  all

rules  and the defuzzified crisp output  from  all

fired rules.

View details about the fuzzy inference

The  above  exercise only allows you to  view  the

combined  result of the output.  GAF  also  allows

you  to  examine the firing details of the  rules.

This  feature along with the stepping mode is  the

best  way  to tuneup or debug the control  system.

From   the  segment  menu,  select  "display  rule

firing",  GAF highlights the first rule  by  using

the  left  column  of the text window.   Use  down

arrow  key to move to rule 5 and press 'Enter'  to

select  it.   Go  back  to the simulation,  you'll

find  the selected rule's fuzzy sets are displayed

to  the  left of the combined result.   Note  that

each  fuzzy  set has the same color  as  the  text

window.   Try  previous  exercises  to  view   the

changes of the fuzzy rule firing status.

Disable rules

GAF  allows  user to disable individual  rule  for

testing  fuzzy  segment.  Let's disable  rule  10,

this  will  effectively break the control  system.

Since  there  is  no  rule  to  respond  when  the

POS_ERROR  is  VERY_LARGE, so after  system  reset

(press  F7)  there  will be  no  output  from  the

control   segment.   From  segment   menu   select

"disable rule", GAF highlights the rule  ID  0  on

the  left of the text window.  Press 'End' to move

the   highlight  cursor  to  rule  10,  and   then

'Enter'.   Now restart the simulation by  pressing

ESC,  F7 then 'F'.  You should see the rule 10  is

marked  with  'X' to indicate it's been  disabled,

and   also  that  there  is  no  response  to  the

position of the robot arm.

Enable rules

There  are  two options under segment  menu  allow

you  to  enable rule(s).  The "enable  all  rules"

option  enables  all rules in this  segment.   The

"enable  rule"  enables the  selected  rule.   So,

type 'A' under segment menu to enable rule 10.

Change sampling rate

Another  important feature in GAF is that you  can

tune  your  system  by trying  different  sampling

rate  (or  cycle  time defined in  each  segment).

From  Option menu select "sim schedule rate",  GAF

prompts  for  new rate. Enter 0.2 (in second)  and

restart  the  simulation.  Please  refer  Schedule

Policy   section   in  GAF.DOC  for   details   of

different schduling mode.

Increment/decrement sampling rate

An  easy way to change the simulated sampling rate

is  to  use the '+' and '-' keys.  Press '+'  each

time  will cause the sample rate to increase 0.005

second,  and  press '-' will decrease  the  sample

rate    by   0.005   second.    The   amount    of

increment/decrement (default is 0.005 second)  can

be  changed  through the Option menu's  "Increment

value"   option.   Now  put  the   simulation   in

continuous  mode by 'C', reset it by F7  and  then

use '+' and '-' keys to adjust the sample rate  to

find  out  what's  the  max  stable  rate  of  the

control loop.

Change output gain

You  can  change  the  gain of  the  output,  from

segment  menu's  "set output  gain"  option.   The

default gain is 1.0.

Check individual segment

This feature allows you to examine one segment  at

a   time.   Let's  examine  the  feedback   loop's

response  at a constant motor current.   Press  F3

once to switch to the feedback segment.  From  run

menu  select  "check segment".  Then from  segment

menu  change the value of input variable "CURRENT"

to  1.0.  Run the simulation by typing 'F' to  see

the response of constant current input.

Getting online help

Press F1 to get online help.

Display the system limits

Press F2 to see the system limits.

View segment information

Press   F4   to   see  the  variable   definition,

initialization  and  normal functions,  and  other

information for the selected display segment.

View symbol definition

Press  F5 to see the fuzzy set symbols' definition

for this segment.

View preset definition

Press  F6  to view the preset definition for  this

segment.

Set new plot time frame

The  default  plot frame period  is  five  seconds

(except  in  sim clock mode), this can be  changed

through display menu's "plot duration" option.

View local variables

The  default display mode is to display  IN,  OUT,

and  INOUT variables, the LOCAL variables are  not

displayed.   Use  "display local variable"  option

in    display   menu   to   enable/disable   LOCAL

variables.

Quit the program

Press   'Alt-Q'   to   quit  the   program   under

simulation or select "quit" option from run menu.

Adapting the Fuzzy Segment

The  genetic  adapt  mode is  an  offline  adaptation

mode.    While   fuzzy  logic  mimics   the   human's

imprecise  reasoning,  the genetic  algorithm  mimics

the  evolution of the nature.  The genetic  algorithm

uses   randomization  techniques  to  reach   optimum

without  exhausting  searching through  the  solution

state   space.  Apply  genetic  algorithm  in   fuzzy

control  eliminates  problems associated  with  fuzzy

system.  GAF uses genetic algorithm to derive  proper

rules  and  fuzzy  sets from the initial  rules.   By

changing,   adding,   deleting   rules   and    fuzzy

membership  sets  of  the fuzzy system,  the  genetic

algorithm  automatically  adapts  and  optimizes  the

fuzzy control system.

The   adapting  example  is  built  on  top  of   the

simulation example, by adding an evaluation  segment.

The   file   "robotarm.evl"  defines  the  evaluation

segment.

Start Adapting

From  system menu, type 'R' to bring up run  menu.

Select  "adapt mode" by pressing 'A'.  GAF  starts

adapting   the  control  loop  and  displays   the

control  segment along with adapt  status  on  the

top of the screen.

Pause every iteration

Press  Alt-P  to  halt  the  adapting  after  each

iteration.  When  an iteration is  completed,  GAF

displays the status (Iteration done: ...)  at  the

bottom  of  the  screen  and  prompts  for  input.

Press 'C' to continue next iteration.  Press  Alt-

C to switch to continuous mode.

Pause every cycle

Press  Alt-A  to  halt  the  adapting  after  each

cycle.  When  a cycle is completed,  GAF  displays

the  status (Cycle done: ...) at the bottom of the

screen  and  prompts  for  input.   Press  'C'  to

continue  next cycle.  Press Alt-C  to  switch  to

continuous mode.

Change size of gene pool

Press  'Esc'  while  in continuous  adapting,  GAF

brings  up the system menu.  Select adapt menu  by

type  'A'.   From  adapt  menu  select  'max  best

items',  and  enter a new pool  size.   This  will

change the number of genes will be saved by GAF.

Change minimum score

Again,  from  adapt menu, select "minimum  score",

GAF  prompts  for new minimum value.   Enter  0.5,

this  tells  GAF to accept all evaluation  results

,which  are  greater  or  equal  than  0.5.    The

default of the minimum score is 0.6.

Change method weight

The  method  menu  provides user  to  control  the

algorithm  of genetic adaptation.  Select  "adding

rule"  from  method  menu, enter  0  for  its  new

weight.   With zero weight, GAF will not  generate

(adding) new rule into the control loop.

Print the result

Select 'File' menu from the system menu, and  type

'A'  to generate report for all genes in the  gene

pool.    The   result  will  be   saved   in   the

adaptlog.rpt, as specified by "report file"  under

File menu, file of the current directory.

Generate statistic report

Again  type  'S'  (generate  statistic  log)  from

'File' menu, GAF produces a statistic report  file

with the file name specified by 'statistic to'  at

the   bottom  of  the  menu.   The  log   includes

statistics   for  all  methods.   Based   on   the

statistic  you  may want to adjust the  weight  of

the methods.